## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Electrical Engg.

Subject: Illumination Engineering(5106)/(EE612) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define White light and CRI. |  |
| b) | State the contrast levels between visual task, working plane, surrounding and field of vision. |  |
| c) | Give three points of distinction between Luminance and Illuminance. |  |
| d) | Define efficacy of a lamp. How is it different from luminous efficiency? |  |
| e) | List out the factors related to interior lighting design. |  |
| f) | Is flood lightning different from interior lightning? Give your choice and justify it giving at least two points of distinction between them. |  |
| g) | What is the significance of silhouetting effect? For which lighting is it applicable? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the factors affecting good lighting. Also state the qualities of good lighting. |  |
| b) | List out the qualities of good lighting systems. Differentiate them with respect to minimisation of hard shadows & luminous efficiency. |  |
| c) | What are luminaires? Classify them. Also give working principle of each of these types and give at least one example of each classified type of Luminaire. | (1+1½+1½+2) |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the types of Candle powers and define each. Also define 1) DLOR 2) ULOR 3) LOR. | (1+2+3) |
| b) | Classify the different lighting sources according to their shapes. A feet florescent lamp is mounted above the front edge of a work bench of 1.5m x 1m size. The mounting height is 1.5m. The transverse intensity of the lamp in lower hemisphere is 600 cd. Calculate the illumination level at a point on the work bench exactly below the edge of the tube. | (2) (4) |
| c) | A lamp having M.S.C.P. of 800 is suspended at a height of 10m. Calculate i) Total flux ii) illumination level just below the lamp. | (6) |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the factors affecting selection of lamps and luminaires in interior lighting design. Also state the factors to be considered in the selection of maintenance factor and give their range of nominal values. -2- | …….2/- |
| b) | An office hall of 20m x 6m x 3.5m is to be provided with general illuminance of 250 lux. Design a suitable illumination scheme using acrylic opal diffuser fitting with 40W, twin tube, 4 feet tube. The relevant data are as given below. Maximum value of S/Hm ratio = 1.5 Frequency of cleaning of fittings = once in two years C.O.U = 0.48 |  |
| c) | List out the salient feature of hospital ward lighting and operation theatre lighting. |  |
| d) | List out the special features of sports lighting. |  |
| Q.No.5. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Classify street light luminaires. Also state the advantages and disadvantages of each type. |  |
| b) | State the criteria to be considered in the selection of mounting height of luminaires used in street lighting. |  |
| c) | State the conditions under which each of the type of street light luminaires can be used. |  |
| d) | Define 1) outreach 2) overhang 3) kerb 4) dual carriage way with the help of neat sketch. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | i) State the essential features of good flood lighting. | (2) |
|  | ii) Define beam spread, beam factor and waste light factor. Also give their nominal values. | (4) |
| b) | A vertical red wall of size 25m x 10m (height) of a monument is to be illuminated from a distance of 30m by projectors planted on the ground. Find the number of fittings, lamps and their arrangement, given that the beam factor is 0.5m, beam spread is 25 (maximum), maintenance factor is 1.6, waste light factor is to be appropriately assumed. Also specify the types of lamps used. |  |
| c) | What is the significance of aiming of flood light projector? Explain it with reference to different values of depth of penetration to the mounting height ratios. |  |

***********