## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Electrical Engg.

Subject: Energy Management(5108)/(EE611) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the importance of flattening of load curve. |  |
| b) | Briefly explain the concept of energy security. |  |
| c) | What is preliminary audit? State its importance. |  |
| d) | What is power factor? Why it should have high value? |  |
| e) | Define pump capacity & hydraulic power. |  |
| f) | What quantities can be computed from no load test on 3 phase induction motor? |  |
| g) | List down the ways by which watt loss in motor can be minimized. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss the quality of incoming supply and its effect on the performance of machinery. |  |
| b) | Write a short note on Energy Conservation Act 2001. |  |
| c) | Write a short note on Energy audit measuring instruments. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe and state advantages and disadvantages of i) Block rate tariff ii) Three part tariff. |  |
| b) | A factory load consist of following: i) Motor of 50 HP rating with 0.8 p.f. lagging and 85% efficiency ii) Synchronous motor of 25HP with 0.9 p.f. leading and 90% efficiency iii) Lighting load of 10KW at unity p.f. Find the annual electricity charges if the tariff is Rs.60 per KVA plus 8 paise per KWh, assuming load to be steady for 2000 hrs in a year. |  |
| c) | Explain need of electrical load management. Briefly explain how better load management can help to minimize peak demand. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain step by step approach for maximum demand control. |  |
| b) | A 3 phase 5KW induction motor has a power factor of 0.75 lagging. A bank of capacitors is connected in delta across the supply terminal and power factor raised to 0.9 lagging. Determine the KVAR rating of the capacitors connected in each phase. |  |
| c) | Briefly explain how motor loading can be determined. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | An office has 45 numbers of 40 watts fluorescent tubelights, length and width of office is 12m & 10m respectively. Ballast of fluorescent tubelight is 15 watts. Mounting height is 2.5m from working plane. Average maintained illuminance is 850 lux. Determine room index, annual energy wastage in KWh, if fluorescent tubelight operates for 5500 hours in a year. Given target lux/w/m2=48. -2- | ……2/- |
| b) | How is pump efficiency determined? Briefly explain the procedure. |  |
| c) | Write short note on automatic power factor controllers. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a short note on soft starter and its advantages. |  |
| b) | What is a ballast? Briefly explain how electronic ballast help in saving energy. |  |
| c) | Explain: - Localised switching - Daylight linked control - Time based control with reference to efficient lighting control |  |

***********