## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Mining Engg.

Subject: Mining Technology(MN302) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Mention the salient features of Jack hammer drill. |  |
| b) | Classify the commercially used drilling methods. |  |
| c) | Give the classification of explosives based on Indian explosive rules. |  |
| d) | Draw a neat sketch of Deck charging. Explain the procedure. |  |
| e) | Write short notes on resin capsule. |  |
| f) | What are the preliminary considerations for locating a shaft? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the surface arrangements in a rotary drilling machine used in opencast mines. Mention the function of each part. |  |
| b) | Describe the construction of a hydraulic prop with a neat sketch. |  |
| c) | What are the different types of roof bolts? Explain any one with a neat sketch. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the salient features of emulsion explosives. |  |
| b) | Write short notes on: i) Plain detonator ii) Booster explosives. |  |
| c) | Enumerate the different properties of an explosive. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the different types of shot holes practiced in underground coal mines? Explain any one with a neat sketch. |  |
| b) | i) What is controlled blasting? Mention the types. ii) Explain the types of secondary blasting. |  |
| c) | On a coal bench of 7m height it was planned to blast 1675 m3 of coal with a burden of 3m and spacing of 4m. How many number of drill holes are required? Also calculate the powder factor of explosive where 25 Kgs of explosive is loaded in each hole. Assume other data if required. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the different methods of shaft sinking adopted in special conditions? Briefly give the steps involved in cementation method. |  |
| b) | Explain the long hole drilling method used in construction of a raise with a neat sketch. |  |
| c) | With a neat sketch describe the temporary lining of a shaft. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain the principle of rotary drilling. What are the different drill bits used? -2- | …..2/- |
| b) | What are the different safety aspects to be considered during transportation of explosives? |  |
| c) | Explain the construction and installation of a cog/chock support. |  |
| d) | Write short note on Ground vibrations due to blasting. |  |

***********