## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Automobile Engg.

Subject: Automobile Chassis-I(AT403) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the basic functions of an Automotive body frame. |  |
| b) | Briefly state the qualities of a good clutch plate lining material. |  |
| c) | Explain Double de-clutching in constant mesh gearboxes. |  |
| d) | What is a Torque tube drive? |  |
| e) | State the points of difference between a Sedan and a Hatchback. |  |
| f) | Why is brake bleeding necessary? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the classification of Automobile chassis with respect to its control. |  |
| b) | With the help of a neat sketch, explain the conventional frame chassis. |  |
| c) | List the various components mounted on the chassis and explain. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in depth the principle of operation of a clutch. |  |
| b) | Draw and explain the working of Diaphragm spring single plate clutch. |  |
| c) | Explain the following briefly: - Multiplate clutch - Clutch pedal play |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the gear shift mechanism in Automotive gearboxes. |  |
| b) | Explain the necessity and functions of an Automotive gearbox. |  |
| c) | With a neat sketch explain the flow of power in top gear in a sliding mesh gearbox. What are the disadvantages of a sliding mesh gearbox? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction of a differential with a neat sketch. |  |
| b) | Draw diagrams showing ‘4x2’ and ‘4x4’. What are the functions of a Transfer case? |  |
| c) | Explain briefly the following: - Three-quarter floating axle - Limited slip differential |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction and working of a master cylinder in Automotive braking system. |  |
| b) | State the properties of an ideal brake fluid. |  |
| c) | With a neat sketch explain the mechanical braking system. |  |

***********