## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Electrical Engg./E&EE

Subject: A.C. Machines(4113)/Electrical Machines-II(EE401) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State three points of difference between slip ring induction motor and squirrel cage induction motor. |  |
| b) | State any two applications of each of the following motors: (i) Synchronous motor (ii) universal motor (iii) Hysterisis motor. |  |
| c) | Compare salient and non salient type of rotor construction of an alternator. |  |
| d) | List the desired conditions for satisfactory operation of alternators in parallel. |  |
| e) | Give the features of synchronous motor. |  |
| f) | Explain the concept of synchronous condenser. |  |
| g) | Explain two bright and one dark lamp method for synchronization of Alternators. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working process of three phase induction motor in detail. |  |
| b) | Draw and explain torque-speed curve and torque–slip curve for three phase induction motor. |  |
| c) | Three phase induction motor has two poles and is connected to 400V, 50Hz supply. Calculate the actual rotor speed and rotor frequency when slip is 4 %. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The power input to rotor of 440V, 50Hz, three phase, 6 pole induction motor is 60KW. It is observed that rotor emf makes 90 complete cycle per minute. Calculate: (i) Slip (ii) rotor speed (iii) rotor copper loss (iv) mechanical power developed. |  |
| b) | Draw a power flow diagram of three phase induction motor and briefly discuss various losses in it. State the expression showing relation between rotor input power, rotor copper loss and mechanical power developed in motor. |  |
| c) | Explain the working of direct on line starter. Draw relevant control circuit diagram. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat control circuit diagram, explain the working of fully automatic star-delta starter. |  |
| b) | Explain the construction and working of shaded pole induction motor. |  |
| c) | State the advantages and disadvantages of stationary armature and rotating magnetic field in alternators. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What do you mean by voltage regulation of alternator? What are the various methods to determine it? Explain any one in detail. -2- |  |
| b) | Explain the effect of variation of excitation on armature current and power factor of synchronous motor. |  |
| c) | A star connected 100KVA, 11KV alternator has rated current of 52.5A. The ac resistance of winding per phase is 0.45ohm. The results are given below: O.C. Test: Field current=12.5A, Voltage between lines =422V S.C. Test: Field current = 12.5A, Line Current =52.5A Determine full load voltage regulation of alternator at 0.8 p.f. lagging. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction and working of Universal motor. |  |
| b) | Explain the construction and working of Repulsion motor. |  |
| c) | Explain the applications of various types of single phase induction motor. |  |

***********