## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Electrical Engg./E&EE

Subject: Utilisation of Electrical Energy(4111)/(EE403) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State any six advantages of electric heating. |  |
| b) | State any four advantages and two applications of infrared heating. |  |
| c) | State any six applications of dielectric heating. |  |
| d) | Briefly explain working principle of high frequency eddy current heating. |  |
| e) | Compare AC & DC arc welding. (any three points) |  |
| f) | Briefly explain the phenomena of arc blow in arc welding. State any two methods of reducing arc blow. |  |
| g) | Define the following with respect to illumination: i) MHCP ii) Brightness iii) Utilization factor |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is indirect resistance heating? Explain with neat diagram, the construction and working of a furnace based on indirect resistance heating. |  |
| b) | With neat diagram explain any two methods adopted for temperature control in resistance heating. |  |
| c) | A 50KW, 3 phase, 400V resistance oven employs nick-chrome strip 0.02cm thick for the 3 phase star connected heating element. If the element temperature is 1000C and that of charge is 700C, estimate suitable width for the element strip. Assume radiant efficiency as 0.7 and emissivity as 0.9. Specific resistance of nichrome alloy is 1.03 x 10-6 Ωm. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Briefly explain the modes of heat transfer. State the expression for heat transfer by each mode. |  |
| b) | With neat diagram explain working and construction of any one induction furnace of your choice. State any two of its advantages. |  |
| c) | Compare resistance welding with arc welding. (atleast six points) |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the need of drooping characteristics in electric arc welding. Also explain the method of achieving it in DC arc welding set. |  |
| b) | Enlist the various current controllers used in welding sources. Describe the construction and working of any one current controller. |  |
| c) | Explain principle of working and construction of any one arc furnace of your choice. |  |
|  | -2- | …….2/- |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in detail the conductor rail system of current collection and its main considerations for electric traction. |  |
| b) | Describe the various essential electrical characteristics of traction motor. Which motor is preferred for traction and why? |  |
| c) | With neat diagrams explain the starting of DC traction system employing 4 motor using series-parallel transition method. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Briefly explain the principle of operation of gaseous discharge lamps. State two of its examples. Also state two each of its advantages and limitations. |  |
| b) | Briefly explain working principle of incandescent lamps. Also compare between incandescent lamp and fluorescent tubes. (atleast three points) |  |
| c) | With neat diagram explain the construction and working of HPMV lamp. |  |

***********