## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Mechanical Engg./A/SB

Subject: Strength of Materials(4032)/(ME401) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define stress, strain and Young’s Modulus. |  |
| b) | State the equation for normal stress and shear stress on a oblique plane at a point in a component where x and y are acting on a material. |  |
| c) | Explain principal stress and principal plane. |  |
| d) | Draw neat sketches of various types of beams. |  |
| e) | Define Moment of inertia and Radius of gyration. |  |
| f) | Explain the theory of simple bending and neutral axis. |  |
| g) | Give the assumptions made in the theory of pure torsion. |  |
|  | only for students of old scheme: |  |
| h) | What is strain energy? What is the relationship for strain energy for gradually applied load and suddenly applied load? |  |
| i) | Explain concept of pure torsion. |  |
| j) | Define Buckling stress and Slenderness ratio. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A steel bar is 800mm long. Its 2 ends are 45mm and 35mm in diameter and length of each rod is 200mm. The middle portion of the bar is 20mm in diameter and 400mm long. If the bar is subjected to an axial tensile load of 20KN, find the total extension and stresses in various sections. Take E = 200 x 103 N/mm2 |  |
| b) | A rod of steel 16m long is at a temperature of 20°C. Find the free expansion of the rod when the temperature rises to 60°C. Also find the temperature stresses produced when i) Expansion of the rod is prevented ii) the rod is permitted to expand by 8mm. Take  = 12 x 10-6/°C and E = 2 x 105 N/mm2 |  |
| c) | A metal bar 60mm in diameter is subjected to an axial compressive load of 500KN. If the contraction of 200mm gauge length was found to be 0.5mm and increase in thickness as 0.05mm, find the value of Young’s modulus and Poisson’s ratio for the bar material. | …….2/- |
|  | -2- |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the shear force and bending moment diagram for the cantilever beam shown in fig.(1). |  |
| b) | Draw the shear force and bending moment diagram for the simply supported beam shown in fig.(2). |  |
| c) | Draw the shear force and bending moment diagram for the simply supported beam shown in fig.(3). |  |
| d) | A load of 100N falls by gravity a vertical distance of 3000mm when it is suddenly stopped by a collar at end of a vertical rod of length 6m and diameter 20mm. The top of the bar is rigidly fixed to a ceiling. Calculate the maximum stress and the strain induced in the bar. Take E = 1.96 x 105 N/mm2 (only for students of old scheme) |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A plane element in a boiler is subjected to a tensile stress of 450 N/mm2 on one plane and 200 N/mm2 on the other plane which is right angles to the former. Each of the above stress is accompanied by a shear stress of 150 N/mm2 such that when associated with the minor tensile stress tends to rotate the element in clockwise direction. Find i) Principal stresses and their directions ii) Maximum shear stress and the direction of the plane on which they act. |  |
| b) | Find out the moment of inertia about XX and YY centroidal axis for a ‘T’ section shown in fig.(4). |  |
| c) | Determine the Moment of inertia of ‘I’ section fig.(5) about XX and YY centroidal axis. |  |
| d) | A shell 3m long, 1m in diameter is subjected to an internal pressure of 1 N/mm2. If the thickness of the shell is 10mm, find the circumferential and longitudinal stresses and maximum shear stress. E = 2 x105 N/mm2 and 1/m=0.3 (only for students of old scheme) |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A rectangular beam 50mm wide and 125mm depth is simply supported over a span of 5m. If the beam is subjected to a central point load of 10KN, find the maximum bending stress in the beam. |  |
| b) | Fig.(5) shows a rolled steel beam of an unsymmetric ‘I’ section. If the maximum bending stress is not to exceed 50 N/mm2, find the moment which the beam can resist. Also determine the safe point load the beam can carry if the I section is simply supported over a span of 6m. -3- | ….3/- |
| c) | A rectangular stud 250mm wide and 200mm thick carries a load of 75KN at an eccentricity of 25mm in a plane bisecting the thickness. Find the maximum and minimum intensities of stress in the section. |  |
| d) | A cantilever of length l = 3m carries a point load of 450N at free end. Find the slope at free end and also the deflection at free end. (only for students of old scheme) |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A shaft is transmitting 120KW at 175 rpm. If the shear stress in the shaft is 65 N/mm2, determine the suitable diameter of the shaft. The shaft is not to be twisted more than 1 in a length of 4m. Take G = 8 x 104 N/mm2 |  |
| b) | Find the diameter of the solid shaft to transmit 120 KW of power at 250 rpm. If the shear stress is not to exceed 85 N/mm2 and the twist shall not exceed 1 in a length of 3.5m. Take G = 8 x 104 N/mm2 |  |
| c) | i) Draw a neat sketch of hollow rectangular section and state the equation for moment of inertia about XX and YY axis. ii) State the equation for torsional stress and strain. Also give its notation. |  |
| d) | Find the shortest length ‘L’ for a pin ended steel column having a cross section of 60mm x 100mm for which ‘Euler’s formula applies. Take ES = 2 x 105 N/mm2 and critical stress limit is 250 N/mm2. (only for students of old scheme) |  |