## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Civil Engg./CT

Subject: Irrigation Engineering(4017)/(CE601) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | How does irrigation help in development of a country/state? Write any 6 points. |  |
| b) | Show the classification of irrigation and mention about each in brief. |  |
| c) | What is meant by Run off? What are the types? Give any two formulae to compute run off. |  |
| d) | Make a list of all the forces acting on a gravity dam. Indicate which of the forces provide stability to the dam. |  |
| e) | What are homogeneous and core type Earth dams? Show neat sketches of each. |  |
| f) | What are the advantages of canal lining? |  |
| g) | Enlist the advantages & disadvantages of bandhara irrigation system. |  |
| h) | What are gates in dams? Sketch & explain any one type. |  |
| Q.No.2. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | What is Hydrologic cycle? Draw a neat sketch showing all the components/phenomena to explain hydrologic cycle. |  |
| b) | Name the three methods of computing average rainfall and explain any one with suitable sketch or numerics. |  |
| c) | Explain with sketch any one type of rain gauge and its working. |  |
| d) | What are the three crop seasons? Name them and mention any one crop in each category. |  |
| Q.No.3. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Define: Duty, Delta, Crop period, Base period, Intensity of Irrigation, Gross Command Area, Culturable Command Area, Capacity factor. |  |
| b) | Enlist the methods to improve duty. |  |
| c) | Water is released at the rate of 30cumec at the head of the canal. If duty at the field is 1500 ha/cumec and loss of water in transit is 20%, calculate the area of land that can be irrigated. |  |
| d) | Draw a neat sketch showing theoretical profile of a gravity dam and label all the forces/parameters. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With clear sketches, derive an expression for ‘No sliding’ condition for a gravity dam. |  |
| b) | i) Enlist any six points to be considered for the best site selection for a dam/reservoir. |  |
|  | ii) Explain the galaries in gravity dams. |  |
| c) | What are the two basic types of earth dams based on method of construction? Explain each in brief. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Fix the T.D.L., N.P.L., L.S.L. for a reservoir referring to the following data. Effective storage required for the crop: 3200 ha-m Tank losses: 20% of effective storage Carry over allowances: 10% of effective storage Dead storage: 10% of gross storage Contours (R.L. in m); 250; 253; 256; 278; 281; 284; 287. Storage (Mm3); 3.3; 4.1; 5.25; 42.65; 47.3; 55.12; 63.00. |  |
| b) | i) What are the three losses in reservoirs? Explain in brief. ii) Enlist at least 6 points on measures to be taken to minimize reservoir losses. |  |
|  | only for students of Revised scheme: |  |
| c) | What are the three classified failures of Earth dams? Explain each in brief specifying further types of failure under each class. |  |
| d) | i) Draw a neat section of a Superpassage. ii) Draw neat sections of canal, one in cutting and one in embankment. |  |
|  | only for students of Old scheme: |  |
| c) | What are the preliminary surveys and data to be collected and the final data to be collected before starting with a new irrigation project. Enlist at least 9 points. |  |
| d) | With a neat sketch (c/s), show the following levels of a reservoir: Bed level, Lowest supply level, Normal pool level, High flood level, Dead storage, Live storage, Surcharge storage. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Spill way and energy dissipaters |  |
| b) | Bandhara irrigation system with sketch |  |
| c) | Canal linings - types based on material |  |
| d) | Aqueduct, Superpassage, Level crossing in canals |  |
| e) | Weirs & Barrages – types and sketches (only for students of Old scheme) |  |

***********