## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Civil Engg./CT

Subject: Quantity Surveying & Costing(4014) [Old]

# Time Duration: 4 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of IS: 7272 (Part1) is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain in brief piece work method of executing work. |  |
| b) | Explain supplementary estimate. When is it done? |  |
| c) | Draw the organisation chart of PWD. |  |
| d) | How is the Engineer’s diary at site maintained by the PWD Engineer? |  |
| e) | Explain lump sum contract. |  |
| f) | State the attributes of a good Valuer. |  |
| g) | Define specifications and state its purpose. |  |
| Q.No.2. | Answer any one of the following Questions: | (15) |
| a) | Fig.1 shows a plan and wall section of a small house. Work out quantities of any five of the following items. Enter them in a standard measurement sheet with full dimensions taken out as per IS:1200. - Earthwork in excavation in foundation - PCC 1:3:6 below foundation and plinth - Laterite masonry in foundation and plinth - Laterite masonry in superstructure - 12mm thick internal plaster - Flooring - vetrified tiles - RCC for roof slab. 12cm thick - Wood work for door and window shutters |  |
| b) | Using suitable method estimate quantity of earthwork between two stations A and B with following data: Width of road 12m; side slope in banking 1.5:1, side slope in cutting 2:1; chain length 30m. Draw longitudinal and cross sectional view. Assume uniformly rising gradient for the new road of 1 in 200. Assume RL of FL at chainage 1 at 101.1m Chainage Point; 1; 2; 3; 4; 5; 6; 7; 8. RL of FL (m); 101.1; 100.75; 99.8; 101.2; 103.2; 103; 103.5; 103.9. |  |
| Q.No.3. | Answer any three of the following Questions: | 3 x 5 = 15 |
| a) | Carry out Rate analysis for laterite stone masonry in cement mortar (1:3) in foundation and basement. |  |
| b) | Carry out Rate analysis for internal plaster of 12mm thick using cement mortar (1:5). |  |
| c) | Explain the terms: - Market value ii) Salvage value iii) Scrap value iv) Potential value e) Book value |  |
| d) | Give five salient features of Indian Contract Act 1872. |  |
| Q.No.4. | Answer any three of the following Questions: | 3 x 5 = 15 |
| a) | Explain the terms: i) EMD ii) Security Deposit. |  |
| b) | Write under what circumstances a tender is rejected. |  |
| c) | Explain how record is kept of the material inventory on site. |  |
| d) | State the different types of specifications and explain any one type in brief. | …….2/- |
|  | -2- |  |
| Q.No.5. | Write short notes on any three: | 3 x 5 = 15 |
| a) | Cubic meter area of approximate estimate |  |
| b) | Essentials of valid contract |  |
| c) | Purpose of estimation |  |
| d) | Contingencies |  |
| e) | Opening of Tenders |  |