## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Engineering & Technology

Subject: Engineering Mechanics(CS301) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What do you meant by angle of repose? How it is related to angle of friction? |  |
| b) | Define force. List down effects of force. |  |
| c) | What is free body diagram? State its use in mechanics. |  |
| d) | Explain Varignon’s theorem. |  |
| e) | State D’Alembert’s principle and give its application. |  |
| f) | Define momentum and impulse. |  |
| g) | What are the assumptions made in analysis of trusses? |  |
| h) | State the principle of conservation of energy. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the resultant of a concurrent force system as shown in fig.(a). |  |
| b) | Find the magnitude and direction of Resultant and its position from ‘B’. Refer fig.(b). |  |
| c) | Non concurrent force system is shown in fig.(c). Find magnitude, direction and location of the resultant force. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A smooth sphere of weight 10N is supported by a chain as shown in fig.(d). The length of the chain AB is twice the radius of the sphere. Draw free body diagram and find the tension in the chain and reaction of the wall. |  |
| b) | Find the reaction at A and D of simply supported Beam shown in Fig.(e). |  |
| c) | Find the forces in the members of truss using method of joints. Refer fig.(f). |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Determine the necessary force ‘P’ acting parallel to the plane as shown in fig.(g) to cause motion to impend. Assume  = 0.25 and pulley to be smooth. |  |
| b) | For the ladder of length 4m shown in fig.(h), determine the minimum horizontal force to be applied to A to prevent slipping.  = 0.2 between the wall and the ladder,  = 0.3 between the floor and the ladder. The ladder weighs 200N and a man weighing 600 N is at 3m from A. |  |
| c) | A mass of 250 gms is dropped on the ground. Just before hitting the ground its velocity is 6 m/s and it rebounds with an initial velocity of 4 m/s. Assuming that the force exerted by the ground (upward) is constant during the period of contact which is 1/40 seconds. Find the value of this force. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A man whose mass is 70 Kg is standing in a lift. What force will the floor of the lift exert on him in the following cases: 1) when the lift stays still 2) when it is ascending with an acceleration of 0.6 m/s2 3) when it is descending with an acceleration of 0.6 m/s2 4) when it is moving with uniform velocity. -2- | …..2/- |
| b) | A person weighing 700N is moving in a light weight passenger elevator weighing 5000N. When the elevator moves upwards with constant acceleration, the cable tension force is 6500N. i) Find the value of the acceleration of the elevator. ii) Find acceleration, if the tension is 4200N as the elevator accelerates downward with a constant value. |  |
| c) | Weights 800N and 400N are connected by a thread and move along a rough horizontal plane under the action of a force of 500N applied to 800N weight as shown in fig.(i). The coefficient of friction between the sliding surfaces of the weights and the plane is  = 0.25. Determine the acceleration of the weights and tension in the thread using the D’Alembert’s principle. |  |
| Q.No.6. | Answer any two of the following QuestionsAny two) | 2 x 6 = 12 |
| a) | A bucket of water weighing 10N is pulled out from a well 20m deep by a rope weighing 0.75 N/m what is the work done? If this work is done in 10 seconds, find power. Length of the rope is 20m. |  |
| b) | A ball of mass 1.2 Kg is dropped from top of a building 20m high. Find its total energy with respect to ground after 1 second. Also, find velocity at ground |  |
| c) | A pump reduces the water level in a well, 4m diameter, by 2m in 20 minutes and lifts the water through an average height of 5m. If the efficiency of the pump is 80%, what power is required to drive the pump. |  |