## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Automobile Engg.

Subject: Automobile Engines-I(4054) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define i) Stroke ii) Clearance volume iii) Compression ratio. |  |
| b) | Explain scavenging in 2 stroke IC engine. |  |
| c) | State the advantages of Post injection in SI engines. |  |
| d) | Sketch the diagram of a simple carburetor. |  |
| e) | Explain detonation in SI engines. |  |
| f) | Draw sketches of any three types of IC engine combustion chambers. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the key differences between 2 stroke and 4 stroke IC engines. |  |
| b) | Draw and explain the working of a Hydraulic Dynamometer. |  |
| c) | The following data refers to a test on a four stroke cycle oil engine: Number of cylinders = 4, cylinder Bore = 10cm, Piston stroke = 14cm, Engine speed = 2400 rpm, brake torque = 185 N-m, fuel oil consumption = 14 litres/hr., calorific value of fuel oil 44000 KJ/Kg, specific gravity of fuel oil =0.83. Calculate i) The brake mean effective pressure (b.m.e.p) ii) The brake thermal efficiency and iii) Brake specific fuel consumption in litre per KW/hr. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction of a Free flow type muffler with a sketch. |  |
| b) | Explain with a sketch i) vibration damper ii) oil pan (sump). |  |
| c) | State the characteristics of Gasoline as a fuel. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain the working of an AC mechanical pump. |  |
| b) | Explain in brief: i) A/F metering in Gasoline engines ii) Choke circuit in a carburetor |  |
| c) | State the advantages of a common rail injection over a conventional direct injection diesel fuel system. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain the construction of a Fuel injector. |  |
| b) | Explain the working of i) Radiator Cap ii) water pump. |  |
| c) | State the necessity of a Thermostat and its working. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the objectives of the lubrication system? Explain in depth. |  |
| b) | Draw a neat sketch and explain the dry-sump type lubrication. |  |
| c) | With a graph, explain in detail the combustion process in a CI Engine. |  |

***********