## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Electrical Engg./E&EE/EI

Subject: Industrial Automation(EN616)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain in brief, the benefits of automation in industry. |  |
| b) | Classify PLCs based on their size. |  |
| c) | State an example of I/O addressing of Allen Bradley PLC and describe the address fields. |  |
| d) | List three points of difference between Ladder programming and instruction list programming. |  |
| e) | Draw a ladder diagram to implement latching and explain the same. |  |
| f) | Describe in brief, the use of PLCs in the railway industry. |  |
| g) | Describe the guidelines followed for proper effective grounding of a PLC for safety and noise reduction. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the block diagram of a PLC and describe the same. |  |
| b) | Explain the function of the I/O image memory and user memory sections of the PLC memory. |  |
| c) | With the help of connection diagrams, explain sinking input and sinking output. Describe two differences between sourcing I/O and sinking I/O. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain three characteristics of each of the following components of a PLC (i) CPU (ii) Power supply. |  |
| b) | Describe three diagnostic features in a PLC. Also explain how redundancy is implemented in a PLC-based system. |  |
| c) | Explain the commissioning procedure for a PLC based system. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe six specifications of the IEC 1131-3 standard. |  |
| b) | With the help of an example, describe structured test programming. |  |
| c) | Draw a ladder diagram to implement the following function: Y = F (a,b,c) =  (0,2,4,6) |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a ladder diagram to implement the following. Two feeder conveyors F1 and F2 feed parts to the main conveyor M. When the ‘NO’ START push button is pressed, F1 is turned ON and feeds parts to M. After feeding five parts, F1 is turned OFF and F2 is turned ON after 10 seconds and remains ON until seven parts are fed to M. When all 12 parts arrive to M, it is turned ON for 30 seconds. ‘NC’ STOP push button is used to stop the process. -2- | …2/- |
| b) | Draw a ladder diagram for temperature control of three heaters H3 by two thermostats T1 and T2, set at 50ºC and 60ºC. At a temperature below 50ºC all three H1, H2 and H3 are ON. At temperature between 50ºC and 60ºC, H1 and H2 are ON and H3 OFF. Above 60ºC all three H1, H2 and H3 are OFF. A start switch turns the system ON and STOP switch turns it OFF. |  |
| c) | Draw and describe a ladder program to control an elevator for three floors and to permit maximum six persons in it at a time. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw ladder diagram to implement the following instructions (i) AND (ii) EXOR (iii) OR, for two inputs. |  |
| b) | Draw a ladder diagram to turn a lamp ON after 30 seconds of pressing the start button, keep the lamp ON for 40 seconds and then turn it OFF. The process repeats until the STOP button is pressed. |  |
| c) | Explain the following ladder diagram instructions with example data: - Greater than - Equal - Less than equal |  |

***********