## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Shipbuilding Engg.

Subject: Basic Ship Theory-I(4331)/(SH301) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short note on any three: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Catamaran |  |
| b) | Length between perpendiculars and moulded breadth |  |
| c) | Displacement |  |
| d) | Body plan |  |
| e) | TRIM |  |
| f) | Moment to cause trim by 1cm (MCT1cm) |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write the salient features of: i) Bulk carrier ii) Barge. |  |
| b) | A ship has displacement 2000 tonnes and KG=4m. She loads 1500 tonnes of cargo (KG=6m), 3500 tonnes of cargo (KG=5m) and 1520 tonnes of fuel (KG=1m). She then discharges 2000 tonnes of cargo (KG=2.5m) and consumes 9000 tonnes of oil fuel (KG=0.5m) during the voyage. Find the final KG on arrival at the port of destination. |  |
| c) | A box shaped vessel 20m x 6m x 2.5m floats at a draft of 1.5m in water of density 1.025t/m3. Find the displacement in tonnes and the height of the centre of buoyancy above the keel. |  |
| Q.No.3. | A ship’s water plane is 72 metres long and the lengths of the half ordinates commencing from the forward are as follows: 0.2, 2.2, 4.4, 5.5, 5.8, 5.9, 5.9, 5.8, 4.8, 3.5 and 0.2m respectively. The spacing between the first three and the last three half ordinates is half of the spacing between the other half ordinates. Find the area of water plane and the LCF. | (12) |
| Q.No.4. | Answer the following Questions: | 2 x 6 = 12 |
| a) | A box shaped vessel is 24m x 5m x 5m and floats on an even keel at 2m draft. KG=1.5m. Calculate the initial meta centric height. |  |
| b) | A ship has KM=6.1m and displacement 3150t. When a mass of 15t already on board is moved horizontally along the deck through a distance of 10m, it causes 0.25m deflection in a 8m long plumb line. Calculate the ship’s KG. (only for students of revised scheme) |  |
| b) | Explain Bonjean curves. (only for students of old scheme) |  |
| Q.No.5. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Explain the types of equilibrium in transverse stability. | . |
| b) | A ship is floating on even keel at 6.0m draft forward and aft. The areas of water planes are as follows: Draft; 0; 1; 2; 3; 4; 5; 6. Area (m2); 5000; 5600; 6020; 6025; 6025; 6025; 6000. Determine the ship’s displacement. |  |
|  | -2- | ……2/- |
| Q.No.6. | Answer any one of the following Questions: | (12) |
| a) | A ship of 6000 tonnes displacement, floating in salt water, has a double bottom tank 20m x 12m x 1.5m which is divided at the centre line and is partially filled with oil of density 0.82. Find the virtual loss of GM due to the free surface of oil? State the ways of minimizing free surface effect. |  |
| b) | A ships water plane is 18m long. The half ordinates at equal intervals from forward are as follows: 0, 1.2, 1.5, 1.8, 1.8, 1.5 and 1.2 metres Find the second moment of the water plane about the centerline and second moment of the water plane about LCF. |  |

***********