## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Mechanical Engg./A

Subject: Hydraulic Machinery(4053)/(ME404) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define surface tension. What are the factors that affect surface tension? |  |
| b) | Define the following: i) Mass density ii) Specific gravity iii) Specific weight. |  |
| c) | Draw a simple sketch showing the relative position of positive gauge pressure, vacuum pressure and absolute pressure. |  |
| d) | State Bernoulli’s theorem. Give its equation stating each head. |  |
| e) | What do you mean by hydraulic gradient line and total energy line? |  |
| f) | Define slip, percentage slip and negative slip in reciprocating pump. |  |
| g) | State the applications of rotary gear pump. |  |
| h) | Give the functions of the following parts of Francis turbine: i) Volute casing ii) Guide vanes iii) Draft tube. (only for students of revised scheme) |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain the working of differential ‘V’ tube manometer. |  |
| b) | Explain the terms ‘pressure intensity’ and ‘pressure head’. Convert pressure head of 15m of water to i) Absolute pressure in KPa ii) Metres of oil of specific gravity 0.8. Take standard atmosphere pressure as 101 KPa. |  |
| c) | A circular plate of 4m diameter is immersed in water in such a way that its greatest depth is 3m below the surface and least depth is 1m below the free surface of water. Find the total pressure and position of centre of pressure. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A rectangular plate 2m wide and 4m deep is immersed vertically in water in such a way that its smaller side coincides with water surface. Find the total pressure and the position of centre of pressure. |  |
| b) | The water is flowing through a pipe having diameters 200mm and 100mm at section 1 and 2 respectively. The rate of flow through the pipe is 35 lit/s. The section 1 is 6m above datum and section 2 is 4m above datum. If the pressure at section 1 is 375KN/m2, find the intensity of pressure at section 2. |  |
| c) | The head of water over the centre of an orifice of diameter 45mm is 1.5m. The actual discharge through the orifice is 0.0056m3/s. Find the coefficient of discharge. |  |
|  | -2- | …….2/- |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A pipe of 250mm diameter and 500m long is discharging water at rate of 0.5 m3/s. Find the head lost due to friction. Take friction factor for the pipe as (4f= 0.02). |  |
|  | only for students of revised scheme: |  |
| b) | Explain the construction and working of Pelton wheel with a neat sketch. |  |
| c) | Explain the phenomenon of cavitation in turbine. What are the causes and effects of cavitation in turbine? |  |
|  | only for students of old scheme: |  |
| b) | With the help of a neat sketch explain construction & working of a vane pump. |  |
| c) | Find discharge over a triangular notch of angle 60 when the head over the notch is 0.2m. Assume cd =0.6. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the different types of losses occurring in centrifugal pump. |  |
| b) | A three stage centrifugal pump is used to deliver 0.05 m3/s of water. Its overall efficiency is 70%. Find the manometer head developed by the pump per stage if the pump is run by 50KW motor. |  |
| c) | Draw a neat sketch of single acting reciprocating pump with air vessel on suction and delivery side of the pump. State the functions of air vessel in a reciprocating pump. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the factor to be considered in the selection of centrifugal pump. |  |
| b) | With the help of a neat sketch explain the construction and working of a hydraulic accumulator. |  |
| c) | Two reservoirs are connected by a pipeline consisting of two pipes in series. One pipe of 150mm diameter is of 6m length and the other pipe of diameter 225mm is of 16m length. If the difference of levels in the reservoirs is 6m, calculate the discharge considering only the head loss due to friction. Take friction factor for both pipes as (4f=0.04). |  |

***********