## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Food Technology

Subject: Food Engineering-I(4265)/(FT304) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any ten of the following Questions: | 10x1.5=15 |
| --- | --- | --- |
| a) | Define thermal conductivity of a substance and give its unit. |  |
| b) | Differentiate between free and forced convection. |  |
| c) | State the features of heat exchangers for gases. |  |
| d) | Define wet bulb temperature and dry bulb temperature. |  |
| e) | What is cooling by evaporation? |  |
| f) | State the principle of vacuum dehydration. |  |
| g) | What is equilibrium moisture content? |  |
| h) | What are the factors affecting storage of dried foods? |  |
| i) | Name the atomizers used in spray dryers. |  |
| j) | State the principle of absorption refrigeration. |  |
| k) | Name two problems of long storage of frozen foods and remedial measures for the same. |  |
| l) | Why are high rates of freezing preferred? |  |
| m) | Draw a material balance for a process concentrating a fruit juice having 8% TSS to a concentrate having 30% TSS. |  |
| n) | What are the applications of steam distillation as an unit operation? |  |
| Q.No.2. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | Explain the concept of thermal resistance in heat transfer by conduction. Hence give an expression for heat transferred by conduction through a three layered composite wall. |  |
| b) | State and explain the law governing radiation of heat, giving the units of the terms therein. |  |
| c) | What are the factors that will improve conduction of heat to a liquid flowing in a pipe in forced convection? Explain. |  |
| d) | Compare plate heat exchangers with tubular-type and list the merits and uses in industry. |  |
| e) | Draw a sketch of scrapped surface heat exchanger, explain its advantages and applications. |  |
| f) | With the help of suitable graphs compare co-current and counter current liquid flow in tubular heat exchangers. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working of any steam heated tubular heat exchanger. Calculate L.M.T.D. in a steam heated tubular heat exchanger where the cold liquid is entering at 35ºC and leaving at 83ºC. State any assumptions you make. |  |
| b) | Draw a neat sketch of a double pipe heat exchanger and state its advantages and specific uses in industry. |  |
| c) | What are humidity charts? How are they used? Calculate the R.H. of air saturated at 30ºC, after it is heated to 65ºC. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a sketch explain multi-effect evaporation and its advantages and applications. What are the advantages and drawbacks of forward and backward feed? | ….2/- |
|  | -2- |  |
| b) | With the help of suitable graphs explain the drying behaviour of food materials. Explain what are the factors affecting drying rates. How can you minimize problems like shape distortion and case hardening? |  |
| c) | Compare the following processes and name products made by each process: i) Spray dryer and drum dryer ii) Vacuum and atmospheric tray dryer. |  |
| Q.No.5. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | Write an explanatory note on freeze-drying and the products for which this processing is suitable. |  |
| b) | Explain the criteria for selection of a dryer for your particular food material. |  |
| c) | Nam three types of concentration equipment, their relative merits and limitations. |  |
| d) | Briefly explain the components and working of a vapour compression refrigeration cycle. |  |
| e) | Write an explanatory note on commonly used refrigerants, their classification and selection. |  |
| f) | Give a brief account of the freezing equipment used in the food industry for freezing fish and fish products. |  |
| Q.No.6. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | Explain the process, advantages and limitations of fluidized bed freezers. |  |
| b) | Explain the difference between slow and quick freezing and the advantages of the latter process. |  |
| c) | Write an explanatory note on the use of low temperature liquids for freezing food materials. Give examples to illustrate your answer. |  |
| d) | What is meant by a ton of refrigeration? Explain how you would calculate the heat load of refrigeration for a particular operation. |  |
| e) | Explain the process of flash vapourization and its applications in the food industry. |  |
| f) | Explain the term fractionation. Briefly describe the equipment used and its working. |  |

***********