## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Architectural Engg.

Subject: Structures-I(4537)/Theory of Structures(AE405) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Differentiate between resolution and composition of forces. |  |
| b) | Define the term “grade of concrete”. Enlist the different grades of concrete in use. |  |
| c) | Enlist the features of load bearing structure. |  |
| d) | Draw the different types of beams with end conditions. |  |
| e) | Differentiate between structural cracks and non-structural cracks. |  |
| f) | Enlist the different types of loads acting on the structure. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the magnitude and direction of the resultant of the following set of forces: - 200N inclined 30 with East towards North - 300N towards North - 350N towards North West - 350N inclined at 40 with West towards South |  |
| b) | Explain the following: - Principle of transmissibility - Parallelogram law of forces |  |
| c) | Find the resultant and magnitude of the following concurrence force system: 150N acting @ 90, 200N acting @ 120, 300N acting at 230, 400N acting at 330. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A simply supported beam of 6m span carries two point loads 10KN and 20KN acting at 2m and 4m from left support respectively. Find the Bending moment and draw B.M.D. and S.F.D. |  |
| b) | A cantilever beam, 3m span subjected to two point loads 5KN and 10KN acting at 2m and 3m from fixed support respectively. Draw B.M.D and S.F.D. |  |
| c) | A simply supported beam of 8m span carries UDL of 10KN/m for entire span and 20KN at the centre. Draw the B.M.D. and S.F.D. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is a framed structure? Explain the following components in detail: i) Column ii) Beams iii) Slabs. |  |
| b) | Find the centre of gravity of channel section whose flanges are 40mm x 10mm and web is 60mm x 10mm. The total height of channel is 80mm. |  |
| c) | State the difference between: - Short column and long column - One way slab and two way slab |  |
|  | -2- | ……2/- |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is a “concrete mix design”? Explain the factors controlling mix design. |  |
| b) | Explain how the following factors helps in prevention of cracks in structures: - Choice of materials - Construction practice and technique |  |
| c) | Explain in detail any two principle causes for occurrence of cracks in structure. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Corbelling in masonry structure |  |
| b) | Solid, Skeleton and Space structure |  |
| c) | Structural and non – structural cracks |  |
| d) | Effects of forces on a body |  |
| e) | Types of internal stresses in a building component and effects |  |

***********