## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Architectural Engg.

Subject: Building Construction-III(AE303) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define foundation. |  |
| b) | What is a framed structure? |  |
| c) | Define Riser and Tread. |  |
| d) | Define Door and Window. |  |
| e) | Differentiate between window and ventilator. Give two most important points. |  |
| f) | “Span is the most important consideration of a timber roof”- Explain. |  |
| Q.No.2. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Sketch out and label all details of an eccentrically loaded foundation with all reinforcement details of R.C.C. construction. |  |
| b) | What is a line out drawing? Explain with a sketch. Also explain the site works executed with it. |  |
| c) | Explain: i) Shoring in excavation ii) Spread foundation |  |
| d) | Differentiate between shallow and deep foundation –give 4 points. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a section though an RCC staircase and label all the details. Draw both the flights. |  |
| b) | Write short notes on: i) Construction joints in RCC ii) Cover in RCC components |  |
| c) | Explain alignment checking in shuttering works of RCC. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the requirement related to dimensions of the following components of a staircase: i) Riser ii) Tread iii) Headroom iv) Width of flight |  |
| b) | Draw and explain: i) Bifurcated staircase ii) Spiral staircase |  |
| c) | Draw in plan and section a single flight steel staircase with all construction details and label it. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw plan and elevation showing all details and labelling of a battened ledged and braced door in timber. |  |
| b) | Sketch and explain: i) Lap joints in timber ii) Tongue and groove joint in timber iii) Fixing of grills to openings |  |
| c) | Draw section and elevation of an Aluminum ventilator showing all details and labelling. | …….2/- |
|  | -2- |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Sketch the elevation of a typical king post roof truss in timber, labelling all its parts and against each part explain the function. |  |
| b) | Explain the construction sequence of a traditional timber sloping roof. |  |
| c) | Sketch and explain: i) Bow string truss ii) Composite truss |  |

***********