## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Mining Engg.

Subject: Surface Mining(MN401) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the permissible limits of bench parameters as per regulations? |  |
| b) | What is scarification? What are the methods employed? |  |
| c) | Define i) Powder factor ii) Controlled blasting. |  |
| d) | Mention the types of shovels used in surface mines. |  |
| e) | Give the reasons for popularity of dumpers in opencast mines. |  |
| f) | List out the equipments/machinery used for construction of dumps. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the stages in design of an opencast mine. |  |
| b) | Enumerate the steps involved in planning process. |  |
| c) | Explain the different cutting methods of a dozer. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | i) How do you select the tips of a ripper? |  |
|  | ii) What are the guidelines for efficient ripping? |  |
| b) | Explain the various components of a typical drilling machine used in opencast mines. |  |
| c) | i) With a neat sketch explain the deck charging method. |  |
|  | ii) What are the different parameters considered in a blast design? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | 6000 T of coal is to be handled daily in 12 hrs of actual working of shovel with expected cycle time to be 30 seconds. The fill factor is 0.8 & swell factor is 1.2. Calculate the bucket capacity of shovel. Assume other data if required. |  |
| b) | What are the different working parameters of a Dragline? How do you calculate the hourly production of dragline? |  |
| c) | Explain the construction and operation of bucket wheel in a bucket wheel excavator? How is the output improved? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the elements which are a part of Haul Road Geometry. |  |
| b) | i) What are the suitable conditions for Belt conveyor? |  |
|  | ii) Calculate the carrying capacity of a belt conveyor travelling at 1 m/s with a cross–sectional area of 0.5m2 (presence of material) and conveying material of density 0.9 T/m3. Assume other data if required. |  |
| c) | Draw a neat sketch of a dump & label different elements/ parts. What safety features need to be incorporated? |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain the cycle of operations in surface mining. |  |
| b) | Give the applicable conditions of a bucket chain excavator. |  |
| c) | What are the different modes of dumper positioning m near a shovel? |  |
| d) | Write a brief note on stabilisation of dumps. |  |

***********