## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Common

Subject: Elements of Mechanical Engineering(3004) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Give the equation, which is used in calculation of power from given Torque and RPM. What are the notations used and their corresponding units? |  |
| b) | What are the main functions of Carburretor? |  |
| c) | State the functions of Blow off cock. |  |
| d) | State the applications of Air-Motors. |  |
| e) | List the important functions of a lubrication system. |  |
| f) | State Bernoulli’s Theorem and write its equation. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | How are Gears classified? Explain Compound Gear Train. |  |
| b) | i) Define slip in belts. ii) Find the power transmitted by a belt running over a pulley of 600mm diameter at 200 rpm, when the tension on tight side is 2500N and tension on slack side is 1244N. |  |
| c) | State advantages and disadvantages of Ball and Roller contact bearings. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat sketch explain working of 4-stroke petrol engine. |  |
| b) | Give six major differences between Petrol engine and Diesel engine. |  |
| c) | Give the functions of the following engine components: - Connecting Rod ii) Crankshaft iii) Camshaft |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare Water tube Boiler with Fire Tube Boiler. |  |
| b) | Classify Air compressors. Describe working of any Rotary compressor with the help of a neat sketch. |  |
| c) | Mention different types of lubricants. State desirable properties of good lubricant. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Enumerate various losses of Head in pipe. How enlargement of pipe increases the loss of Head? |  |
| b) | What is Priming? Explain with neat sketch the construction and working of centrifugal pump |  |
| c) | What is cavitation? How does it affect the working of pumps? |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Economiser |  |
| b) | Open belt drive and cross belt drive |  |
| c) | Classification of I.C. Engines |  |
| d) | Equation of continuity |  |

***********