## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Automobile Engg.

Subject: Machine Tools & Production Processes-II(4060) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw a neat labelled sketch of a milling machine arbor. |  |
| b) | Briefly explain the utility of a boring bar and boring head. |  |
| c) | List the advantages of a hydraulic shaper mechanism. |  |
| d) | Differentiate between Puncher slotter and Precision slotter. |  |
| e) | What are the various work holding devices used on a planer? |  |
| f) | Draw a neat sketch of a Radial Drilling Machine. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the construction and working of a Gang drilling machine. |  |
| b) | An 10mm  hole is to be drilled on a material having cutting speed of 25 m/min. Calculate the optimum spindle speed. |  |
| c) | Explain the following operations on a drilling machine with neat sketches: (1) Spot facing (2) Reaming (3) Boring. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the main parts of a horizontal column and knee type milling machine. |  |
| b) | Explain the difference between Up milling and Down milling process. |  |
| c) | Calculate the machining time required to complete a cut, given the following data: (1) Length of table travel = 260mm (2) Feed per tooth = 0.1mm (3) No. of teeth on cutter = 40 (4) R.P.M. of the spindle = 450 |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Why is quick return required on a shaper? How is it achieved hydraulically? Explain. |  |
| b) | Explain the following operation on a shaper: (1) Machining of inclined surfaces (2) Machining of flat horizontal surface |  |
| c) | With a neat sketch explain the main parts of a standard shaper. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the open and cross belt drive of a planer. |  |
| b) | Draw a neat labelled sketch of a slotter and explain its working. |  |
| c) | Explain how T-bolt and clamps are used to hold a work piece on a milling machine table with a neat sketch. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Vertical boring machine operation |  |
| b) | Double housing planer |  |
| c) | Jig boring machine |  |
| d) | Slotter operations |  |
| e) | Size of horizontal boring machine |  |

***********