## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Architectural Engg.

Subject: Building Construction-I(AE105) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five in short: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List out any 3 building components in the sub-structure of a building. |  |
| b) | Name 3 different building materials used in sloping roofs in Goa. |  |
| c) | Give 3 reasons why drawings are drawn to scale. |  |
| d) | Name and sketch one tool each of the: - Painter ii) Mason iii) Carpenter |  |
| e) | Give 3 points of difference between a brick wall and a brick pier. |  |
| f) | Sketch a standard first class brick with all its features. Mention at least 3 dimensions and at least 4 labelling parts. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Sketch a Semi-circular Arch and label atleast 8 of its important parts. |  |
| b) | Write a note on the following aspects of wall as a building component: - Function - explain atleast 3 - Materials – explain atleast 3 |  |
| c) | Write a short description of the following openings: - Doors ii) Windows iii) Ventilators |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain various characteristics of a i) Lean-to-roof ii) Pyramidal Roof. |  |
| b) | Name any 6 different building materials and sketch their graphical representations. |  |
| c) | Name and sketch any 6 different signs and symbols used in an Architectural Drawing. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Sketch out the following equipment used in construction works: - Vibrator ii) Mixer iii) Grinder iv) Roller v) Plumb bob vi) Drill |  |
| b) | Explain any 2 equipments/Machines used in excavation works done mechanically. |  |
| c) | A Balcony is to be built with a parapet wall on one side in bricks. Which brick bond would you suggest? Draw the required details to scale explaining its construction with necessary labelling and dimensions. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain 6 characteristics of a Flemish Bond. |  |
| b) | Draw the alternate plans and part elevation with dimensions and labelling to suitable scale to explain 1½ brick English Bond. -2- | ……2/- |
| c) | Which brick bond would you suggest for the following wall construction and give one important reason why: - Chimney over the roof top - Partition wall in the room - Exposed Brick compound wall - Load bearing house wall |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain lintels and suggest 2 construction materials used in its making. |  |
| b) | Write out the main work handled on construction sites by the following: - Fitter ii) Gawandi iii) Bai iv) Chowkidar |  |
| c) | Sketch out any 4 tools used by a labourer in construction works. |  |
| d) | Explain any 2 other types of bricks (other than the standard brick type) available in the market. |  |

***********