## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Electronics Engg./EC/EI/MED/E&EE

Subject: Circuits & Networks(4136)/(EN403) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Calculate the value of C required to obtain an equivalent capacitance of 4µf across terminals A-B of fig.1a. |  |
| b) | State the voltage divider theorem. Calculate the voltage across the 6Ω resistor of fig.1b. |  |
| c) | State Kirchoff’s current law and explain it with the help of a simple circuit. |  |
| d) | For a sinusoidal voltage, write equations and describe the phase difference between the input and output voltage for a circuit with i) a resistor ii) a capacitor. |  |
| e) | Describe the formula for Zsc of a symmetrical T network. |  |
| f) | Write the equation describing the relationship between decibel and neper. Convert 2dB into nepers and 5 nepers into dB. (only for students of revised scheme) |  |
| f) | Write a short note on RC integrator. (only for students of old scheme) |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Determine the magnitude of current through the 4Ω resistor of fig.2a and the voltage across it. Use mesh analysis method. |  |
| b) | Calculate the current through the 2Ω resistor of fig.2b, using superposition theorem. |  |
| c) | Compute the equivalent resistance between terminals A and B of fig.2c. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Determine the magnitude of current and its direction through resistor R of fig.3a. Use nodal analysis method. |  |
| b) | Calculate the Thevenin’s equivalent voltage VTH and resistance RTH between terminals AB of fig.3b. Also, calculate the voltage across the load resistor RL. |  |
| c) | Calculate the value of R which will absorb maximum power from the circuit of fig.3c. Also, calculate the maximum power absorbed by R. |  |
|  | -2- | …..2/- |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw an RC differentiator circuit and the input and output waveforms for sine and square wave inputs. Write the equation for the output corresponding to a sinusoidal input and describe it. |  |
| b) | Draw a series RLC circuit R=10Ω, L=1mH, C=1µf and applied voltage of 200V. Write equations for impedance z and current i and explain with the help of a resonance curve, how i and z change at various frequencies. |  |
| c) | For a series resonant circuit using R=20Ω, L=100mH and C=10µf, calculate i) Resonant frequency ii) Bandwidth iii) Half power frequencies. |  |
| d) | Explain the concept of active power and reactive power in a circuit. (only for students of old scheme) |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A symmetrical  network has a 1KΩ resistor in its series arm and 2KΩ resistors in the shunt arms. Calculate i) Characteristics impedance ii) Short circuit impedance iii) Open circuit impedance. |  |
| b) | i) Describe image impedance of a network. What is its significance? ii) Calculate open circuit voltage and voltage across a load of 10Ω when a voltage source of 10V with internal resistance of 0.1Ω is connected to it. |  |
| c) | Calculate the image impedances and propagation constant of the network shown in fig.5c. (only for students of revised scheme) |  |
| c) | Explain the transient response in a simple RL circuit with a circuit diagram and necessary waveforms. (only for students of old scheme) |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a low pass filter circuit and draw and describe its frequency response. Calculate the cut off frequency of a LPF using R=2KΩ, C=10µf. |  |
| b) | Design a symmetrical T attenuator circuit using resistors, to provide an attenuation of 20dB and characteristic impedance of 450Ω. |  |
| c) | i) Design a constant –k HPF (T type) for a design load impedance of 600Ω and cut off frequency of 4 KHz. ii) Distinguish between linear and non-linear circuit elements. List two examples of each. |  |
|  |  | ……3/- |