## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Civil Engg./CT

Subject: Transportation Engineering-I(CE304) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw a neat cross section of a Road in full cutting of a National Highway. |  |
| b) | Define (i) Camber (ii) Spoil Banks (iii) Gradient. |  |
| c) | Distinguish between Tack coat and Seal coat. |  |
| d) | Write a short note on ‘Importance of Transportation System in National Development’. |  |
| e) | What are ‘Protective works’ in hill Roads? |  |
| f) | Write a short note on ‘Selection of trees for arboriculture’. |  |
| g) | What is a Causeway? How is it different from a Culvert? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat sketches explain how superelevation can prevent a vehicle from overturning. |  |
| b) | Explain the step by step procedure of construction of a WBM Road. |  |
| c) | Explain two methods of soil stabilisation of Earth Roads. Also state why soil stabilisation is required. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in detail the various defects in Bituminous concrete roads. How are these roads maintained? |  |
| b) | Explain ‘Stoppping sight distance’ and ‘Overtaking sight distance’ with respect to hill road geometric design. Also mention stopping sight distance and overtaking sight distance for different speeds as recommended by IRC. |  |
| c) | How is sub surface drainage done for roads? What method is adopted to control capillary rise? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Distinguish between Flexible and Rigid pavements. |  |
| b) | With neat sketches explain the three types of transverse joints in concrete roads. |  |
| c) | What are the various layers (component parts) of a concrete road? Explain in detail the process of i) Floating and straight edging ii) Belting, booming and straight edging while constructing concrete roads. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch of ‘Clover leaf Flyover’ and state its uses. |  |
| b) | What are the functions of bearings in bridges? With a neat sketch explain any one type of a fixed bearing. |  |
| c) | Explain with a neat sketch ‘Steel Suspension Bridge’. Also state its advantages and suitability. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | IRC classification of roads |  |
| b) | Types of traffic signs |  |
| c) | Necessity of Traffic studies |  |
| d) | Box culvert |  |
| e) | Piers, Abutments & Wingwalls |  |

***********