## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Electronics Engg./EC/EI/COMP/MED

Subject: Microprocessors(PD&I)-II(4147) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Which are the I/P handshaking signals of 8255? Explain in brief. |  |
| b) | Draw block diagram of 8155. |  |
| c) | Specify the features of 8253 (at least 3). |  |
| d) | Why each channel of DMA controller can access only 16KB of memory? |  |
| e) | What do you mean by fully nested mode and special fully nested mode of 8259? |  |
| f) | What do you mean by left entry and Right entry of display of IC 8279? |  |
| g) | What do you mean “Bus standard”? Give two features of RS-232 bus standard. |  |
| h) | Explain in brief traffic light controller. |  |
| Q.No.2. | Answer any two the following Questions: | 2 x 6 = 12 |
| a) | Explain BSR mode of 8255 in detail with two examples. |  |
| b) | Which are the timer modes of 8255? Explain with neat waveforms. |  |
| c) | Draw output waveforms of all the modes of 8253 if counter 1 is loaded with count=4. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain bidirectional mode of 8255 with suitable example. |  |
| b) | Explain Initialization Control Words (ICWS) of IC 8259. |  |
| c) | Explain the functions of following pins of DMA controller 8257: i) AEN ii) ADSTB iii) TC iv) Mask |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw block diagram of IC 8251 & explain it. |  |
| b) | Explain the working of IC 8259 Interrupt controller. |  |
| c) | Explain keyboard section of 8279 in detail. |  |
| Q.No.5. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain the working of Laser printer. |  |
| b) | Explain the difference between floppy disc and hard disc (at least 4 points of differences). |  |
| c) | Explain working of GPIB Bus standard. |  |
| d) | Explain the working of Hall effect keys & membrane keys. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain working of CRT. |  |
| b) | Differentiate between FIFO RAM and Display RAM (at least 4 points). |  |
| c) | Explain Operational Control Words (OCWS) of 8259. |  |
| d) | Explain the working of cascading lines CAS0, CAS1 & CAS2 of IC 8259. |  |
| e) | Explain the working principle of any one type of Printer. |  |

***********