## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Shipbuilding Engg.

Subject: Basic Ship Theory-II(SH401)/(4335) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short notes on any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Capacity plan |  |
| b) | Angle of down flooding |  |
| c) | Wall sided formula |  |
| d) | Pivotting and tipping |  |
| e) | Lost bouyancy method |  |
| f) | Load curve and Shear force curve |  |
| g) | Bale and grain capacity |  |
| Q.No.2. | A box shaped vessel 150m long, 20 beam is floating upright in salt water at drafts of 6m forward and aft. The collision bulkhead is situated 8m from forward. Find the new drafts if the vessel is now bilged forward of the collision bulkhead. KG =2.5m | (12) |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a set of typical end launching curves. |  |
| b) | A ship of 10,000 tonnes displacement has righting levers as follows. Heel; 0; 10; 20; 30; 40. Gz(m); 0; 0.09; 0.21; 0.30; 0.33. Calculate the dynamic stability to 40 degrees heel. |  |
| c) | A box shaped vessel 30m x 6m x 4m floats in salt water on an even keel at 2m. KG=3m. Calculate the angle of loll. |  |
| Q.No.4. | A box shaped vessel 100m long floats on an even keel displacing 2000 tonnes. The mass of the vessel alone is 1000 tonnes evenly distributed and she is loaded at each end for a length of 25m with 500 tonnes of cargo, also evenly distributed. Sketch the curves of load, shear force and bending moment. | (12) |
| Q.No.5. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Explain the weight curve and bouyancy curves drawn in longitudinal strength analysis. |  |
| b) | Explain briefly the longitudinal strength of a ship. ### OR |  |
| Q.No.5. | From the section shown in fig.1, calculate i) Position of the neutral axis ii) Moment of the inertia about the neutral axis iii) Section modulus at deck and keel. | (12) |
|  | (Only for students of revised scheme) |  |
| Q.No.6. | The righting levers of a ship, for an assumed KG=3.5m are 0, 0.25, 0.46, 0.51, 0.39, 0.10 and – 0.38m at angles of heel of 0, 15, 30, 45, 60, 75 and 90 respectively. Draw the amended curve of statical stability when the ship is loaded to the same displacement and the centre of gravity is 3.0m above the keel. | (12) |
|  | (Only for students of old scheme) |  |
| Q.No.6. | A vessel of 8000 tonne displacement was inclined by moving 5 tonne through 12m. The recorded deflection of a 6m pendulum is 78 mm. If the KM for this displacement was 5.10m, calculate KG of the vessel. | (12) |