## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Mining Engg.

Subject: Underground Mining Methods(MN402)/(4296) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain Bord and Pillar method of development. |  |
| b) | Sketch and label Hydraulic Stowing Plant layout. |  |
| c) | State advantages and disadvantages of Longwall retreating method. |  |
| d) | Define thick seam and classify seam on the basis of thickness. |  |
| e) | Sketch and label cut and fill stope. |  |
| f) | Enumerate advantages and disadvantages of caving methods. |  |
| Q.No.2. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Explain preparatory arrangement before depillaring. |  |
| b) | A coal seam 2.5m thick at depth 175m from surface dips at l in 7 is worked with pillar size 35m x 30m centre to centre and gallery size 4m wide x 2.5m height during development. During depillaring split gallery 4m wide are driven. Find i) Percentage extraction during development ii) Production per shift if 4 splits and 3 stooks are worked with one meter per shift advance. Assume specific gravity of coal is 1.3. |  |
|  | ### OR |  |
| b) | Enumerate advantages and disadvantages of hydraulic stowing. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain shearer sumping methods. |  |
| b) | With sketch explain Longwall advancing method. |  |
| c) | Explain method of material handling and drilling in metal mines (underground). |  |
| Q.No.4. | Answer the following Questions: |  |
| a) | State application of block caving method. | (3) |
| b) | Enumerate methods of thick seam mining. | (3) |
| c) | With sketch explain sub level stoping. | (6) |
|  | ### OR |  |
| c) | Compare cut and fill method with shrinkage method. | (6) |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With sketch explain over hand and under hand stoping. |  |
| b) | Enumerate application conditions, advantages and disadvantages of square set stopping method. |  |
| c) | Explain factors affecting choice of metal mining methods (stoping). |  |
| d) | List out machinery used in Longwall mining and state their purposes. |  |
| Q.No.6. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Explain thick seam working by Longwall method. |  |
| b) | With sketch explain sub level caving method. |  |
|  | OR |  |
| b) | Explain methods of working of a narrow vein. |  |

***********