## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Mechanical Engg.

Subject: Manufacturing Process-III(4046) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Write a note on Edge planer. |  |
| b) | List the different types of broaching machines used in industry. |  |
| c) | What is meant by pull and push broach? |  |
| d) | Explain the principles of gear generating process. |  |
| e) | Draw knurling tool holder. |  |
| f) | What is overcut in EDM? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the following planing operations: i) Planing vertical surface ii) Planing a slot. |  |
| b) | State the rule of compound indexing. Index 69 divisions using compound indexing. |  |
| c) | Describe following tool holding devices giving suitable sketches: i) Multiple cutter holder ii) Straight cutter holder. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the methods used for broaching? Briefly explain each method. |  |
| b) | With the help of neat sketch explain the gear manufacturing process by using formed end mill cutter. |  |
| c) | Explain the Bar feeding mechanism in capstan and turret lathe. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the process of boring with neat sketch. |  |
| b) | With the help of sketch explain the construction and use of simple dividing head. |  |
| c) | Differentiate between capstan and turret lathe. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Giving neat sketch explain the process of laser beam machining. |  |
| b) | With the help of neat sketch explain the process of Gear hobbing. Also state its advantages. |  |
| c) | Explain the different steps involved in the production of hexagonal bolt on a capstan and turret lathe. |  |
| Q.No.6. | Write short note on any three: | 3 x 4 = 12 |
| a) | Double housing planer |  |
| b) | Boring Heads |  |
| c) | Continuous Broaching |  |
| d) | ECM process |  |

***********