## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Mechanical Engg./A

Subject: Machine Drawing-I(4033) [Old]

# Time Duration: 4 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | The projectors of the ends of a line AB are 50mm apart. The end A is 20mm above the H.P. and 30mm infront of the V.P. The end B is 35mm above the H.P. and 60mm infront of the V.P. Draw the projections of the line. Determine its true length and inclinations with the H.P. and the V.P. using auxiliary projection method. |  |
| b) | A regular hexagon of 40mm side has a corner in the H.P. Its surface is inclined at 45 to the H.P. and the top view of the diagonal through the corner, which is in the H.P, makes an angle of 60 with the V.P. Draw its projections using auxiliary projection method. |  |
| c) | Fig.1 shows the front view and the top view of a machine part. Draw the auxiliary top view of the apart showing the true shape of the inclined surface. |  |
| Q.No.2. | Answer any one of the following Questions: | (12) |
| a) | A vertical square prism, base 50mm side has its faces equally inclined to the V.P. It is completely penetrated by another square prism of base 30mm side, the axis of which is parallel to both the planes and is 6mm away from the axis of the vertical prism. The faces of the horizontal prism are also equally inclined to the V.P. Draw the projection of the solids showing lines of intersection. Assume the length of the prisms as 100mm. |  |
| b) | A vertical cylinder of 60mm diameter is penetrated by a square prism of 35mm side, the axis of which is parallel to both the planes and is 10mm infront of the axis of the cylinder. The faces of the prism are equally inclined to the V.P. Draw the projection of the solids showing the curves of intersection. Assume the lengths of both the solids as 100mm. |  |
| Q.No.3. | Answer any one of the following Questions: | (10) |
| a) | A square prism of base 50mm side and height 125mm stands on the H.P. on its base with a side of the base inclined at 30 to the V.P. It is penetrated by a cylinder of 50mm diameter and 125mm long, whose axis is parallel to both the H.P. and the V.P and bisects the axis of the prism. Develop the lateral surface of the prism. |  |
| b) | Fig.2 shows the front view of a 45 elbow. Develop the lateral surfaces of the two portions of the elbow. |  |
| Q.No.4. | Fig.3 shows the front view and left side view of a machine part according to first angle method of projection. Draw the following according to the same method of projection: - Sectional front view (section A-A) - Left side view - Top view | (14) |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.5. | Answer the following Questions: |  |
| a) | Give the conventional representation for the following: - Ball bearing ii) Rubber iii) Concrete iv) Stud | (09) |
|  | OR |  |
| a) | Show the following with the help of suitable freehand sketches: i) Spoked wheel in section ii) Full section iii) Partial section iv) Removed section. | (09) |
| b) | Draw the double line convention of the following pipe fittings: i) 90 elbow ii) check valve iii) Cross. | (09) |
| Q.No.6. | Answer any one of the following Questions: | (09) |
| a) | Fig.4 shows the orthographic views of a machine part to be fabricated by welding. Redraw the same and indicate suitable welds using appropriate symbols. |  |
| b) | Draw the sectional representation and symbols for the following welds: i) Single U butt weld ii) Double V butt weld iii) Spot weld. | …..3/- |

+