## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Mechanical Engg./FT/A/SB/MNE

Subject: Elements of Electrical & Electronics Engineering(CS302) [Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw neatly a sinusoidal A.C. voltage waveform and label Time period and maximum value of voltage. |  |
| b) | State any 3 methods used to decrease the earth resistance. |  |
| c) | State the names of 2 types of Fuses and also draw their symbols. |  |
| d) | Write the E.M.F. equation of Transformer and state each term in the equation. |  |
| e) | Classify D.C. Motors and state one application of each Motor. |  |
| f) | State any two parts of squirrel cage Induction Motor and state their functions. |  |
| g) | Draw a neat labelled diagram of centre-tap Full wave rectifier. Also draw input and output waveforms. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List the specifications of P.V.C insulated cable. Compare surface conduit wiring system and concealed wiring system. |  |
| b) | With a neat single line diagram, explain the various stages of distribution of power. Also state the voltage level at each stage. |  |
| c) | With neat diagram, explain the construction of 3-phase P.V.C. insulated cable. State the function of each part of cable. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | i) Draw the symbol of switch fuse unit and explain its working. ii) State the colour codes of 3-phase P.V.C cables. |  |
| b) | Write a note on ‘Limit Switches’. |  |
| c) | State the functions of MCB, ELCB, Contactor, Electromagnetic Relay. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat diagram, explain the principle of operation of a single-phase Transformer. |  |
| b) | List the losses in Transformer core and windings. Explain the losses in transformer and how they can be decreased. |  |
| c) | With neat circuit diagram, explain the working principle of D.C. Motor. |  |
| d) | Explain the reversal of direction of rotation of D.C. shunt Motor and D.C. series Motor. (Draw neat diagram) |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the necessity of starter for 3-phase Induction Motor. State the names of starters used for 3-phase Induction Motors. |  |
| b) | Explain the operation of 3-phase Induction Motor. (Draw neat diagram) |  |
| c) | Draw the symbols and write the truth tables of the following gates: i) NAND ii) EX-OR iii) NOR. -2- | …..2/- |
| d) | Convert the following decimal number to binary: 48 |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw neat circuit diagram of Bridge Rectifier. Explain its working and also draw input and output waveforms. |  |
| b) | What is Extrinsic Semiconductor? State any 2 types of extrinsic semiconductors. Explain the formation of any one type of extrinsic semiconductor. |  |
| c) | With neat circuit diagram, explain the working of NPN transistor. |  |

***********