## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Electrical Engg./E&EE

Subject: D.C. Machines(4109) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List the various parts of DC Machine. |  |
| b) | Explain the concepts of single layer and double layer winding in a D.C. Machine. |  |
| c) | Write the emf equation of dc generator and state the factors on which it depends. |  |
| d) | Explain the construction of a bipolar dc welding generator. |  |
| e) | What are the conditions to be fulfilled for parallel operation of DC generator? |  |
| f) | Explain the principle of operation of d.c. motor. |  |
| g) | What is the necessity of starter in D.C. motor? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a diagram of simple loop generator and explain its working in brief. |  |
| b) | With a neat diagram explain the constructional features of third brush welding generator. |  |
| c) | Two stunt generators running in parallel have each an armature resistance if 0.02 and a field resistance of 50. The combined external load current is 5000A. The fields are excited so that the emf induced in one machine is 600V and in second machine is 610V. Calculate the bus-bar voltage and output of each machine. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a note on various losses occurring in a dc generator. |  |
| b) | Draw the external characteristics of a d.c. shunt generator and explain it in brief. |  |
| c) | A 4 pole d.c. shunt generator with wave connected armature has 32 slots with 16 conductors per slot. The flux per pole is 25 mwb. If the armature is driven at 1000 rpm, calculate the emf induced. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the methods used for speed control of d.c. series motor and explain any one in detail with the help of neat diagram. |  |
| b) | Draw and explain speed and torque characteristics for D.C. series motor. Also give applications of D.C series motor. |  |
| c) | With a neat diagram explain construction and working of 3 point starter. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare shunt, series and compound motors with respect to starting torque with the help of characteristics. |  |
| b) | Write short notes on: - Importance of back emf in DC motor - Losses occurring in DC motor |  |
|  |  | …..2/- |
|  | -2- |  |
| c) | A 250V, 4 pole, wave wound d.c. series motor has 782 conductors on its armature. It has armature and series field resistance of 0.75. The motor takes a current of 40A. Estimate its speed and gross torque developed if it has a flux per pole of 25 mwb. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List the different methods for improving commutation process. Explain any two methods. |  |
| b) | With neat diagram explain the concept of armature reaction and its effect on operation of D.C. machine. List the methods that can be adopted to reduce each of these effects. |  |
| c) | State the conditions required to be satisfied by a D.C. shunt generator to build up the maximum voltage. State current and voltage equations for i) D.C. shunt generator ii) DC series generator. |  |

***********