## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Automobile Engg.

Subject: Automobile Chassis-II(4062) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Discuss the criteria for stability of a vehicle on slopes. |  |
| b) | Explain what is “Tractive Effort”. |  |
| c) | Briefly explain the terms Rolling, Pitching and Bouncing. |  |
| d) | What is Aerodynamics coefficient? |  |
| e) | Explain the purposes of steering system. |  |
| f) | Explain the use of a Torsion Bar with a neat sketch. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the term “Drawbar Pull”. How is it calculated? |  |
| b) | Explain the term “gradiablity” and how it is calculated. |  |
| c) | A motor car weighs 18.14 KN including the four road wheels each of which has an effective diameter of 66 cm, a radius of gyration of 28 cm and a weight of 300N. It has an engine developing 67.5 KW at 2400 rpm. The parts which rotate at engine speed weigh 1090N, with a radius of gyration of 11.5 cm. The transmission efficiency is 90% and the total Road and air resistance at this engine speed in top gear of 3.84:1 is 890N on level. Calculate the acceleration in m/sec2, under these conditions and assuming the acceleration to be uniform, the time required to increase the speed by 32 Km/hr. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with sketches “Toe-in and Toe-out” and “Caster”. What are their effects on proper steering? |  |
| b) | With the help of a neat sketch, explain the working of a hydraulic rack and pinion type of power steering. |  |
| c) | Using a neat sketch explain the working of a Davis steering mechanism. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain what is “Overdrive” and its working. |  |
| b) | Explain with a neat sketch the working of a Torque converter. |  |
| c) | With neat sketch explain the working of one type of Automatic transmission. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the Double wishbone type of suspension for a car. |  |
| b) | With neat sketch describe the telescope shock absorber. |  |
| c) | Using neat diagram, explain the leaf spring suspension for a beam axle. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the various rust inhabiting techniques to prevent corrosion in Automobiles. |  |
| b) | Explain with a neat sketch the door lock mechanism of a car. |  |
| c) | Compare between Integral chassis and Framed chassis structures. |  |

***********