## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Electrical Engg./EI

Subject: Circuit Theory(EE301)/(4104) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define linear, non-linear and bilateral circuit. |  |
| b) | For the circuit shown in fig.1, find the equivalent resistance across terminals A & B. |  |
| c) | Derive the expression for the power consumed in a R-L circuit. Draw the waveform diagram for the power. |  |
| d) | A capacitor and resistor are connected in series to an ac supply of 50V, 50Hz. The current is 2A and power dissipated is 80W. Calculate the resistance and capacitance of the capacitor. |  |
| e) | Define positive, negative and zero sequence components. |  |
| f) | Draw Hay’s bridge circuit. List down its advantages and disadvantages. |  |
| g) | Draw De Santy bridge circuit. Give its equations at balance. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | For the circuit shown in fig.2, determine the current in each branch by using nodal analysis. |  |
| b) | Find the current through RL by using Thevenin’s theorem for the circuit shown in fig.3. |  |
| c) | Find the potential across 2 ohm by mesh method for the circuit shown in fig.4. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A coil of resistance 20 ohms and inductance of 0.04 henry is connected to an ac supply of 230V, 50Hz. Calculate i) Current ii) power factor iii) voltage drop across resistance & inductance & iv) power consumed in the circuit. |  |
| b) | A circuit takes a current of 3A at 0.6 p.f. lagging when connected to a 115V, 50Hz supply. Another circuit takes a current of 5A at 0.707 p.f. leading when connected to same supply. If the two circuits are connected in series across 230V, 50Hz supply, calculate the current drawn, power consumed & p.f. |  |
| c) | Calculate the supply voltage, active power, reactive power and the apparent power in series circuit as shown in fig.5. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A coil having an inductance of 50mH and resistance of 10 ohm is connected in series with a 25µF capacitor across a 200V ac supply. Calculate i) resonant frequency of the circuit ii) current flowing at resistance and iii) value of Q0. -2- | ……2/- |
| b) | A parallel resonance circuit has a coil of 150mH with a Q0 of 60 and is resonated at 1MHz. - Calculate the value of required capacitance. - What is the resistance of the coil? - What is the impedance at resistance? - If Q0 is reduced by 4 adding additional series resistance, how much resistance is needed? - What is the antiresonance frequency with new Q0? |  |
| c) | Explain the resonance frequency, Q factor and bandwidth with neat diagram. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Three similar coils each having a resistance of 20 ohms and inductance of 0.05H are connected in star to a 3-phase, 50Hz supply with 400V between lines. Calculate the total power absorbed and line current. Draw the vector diagram of currents and voltages. |  |
| b) | Three identical impedances are connected in delta to a 3-phase, 400V, 50Hz supply. The line current is 35A and the total power taken from the supply is 14.5KW. Calculate the resistance & inductance of each impedance. |  |
| c) | A star connected load having R=42.6 ohms/phase and XL=32 ohms/phase is connected across 400V, 3-phase supply. Calculate i) line current, reactive power and power loss & ii) line current when one of the load becomes open circuited. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the positive, negative and zero phase sequence components of the following set of three unbalanced voltage vectors: VA=1030, VB=30-60 & VC=15145 volts. |  |
| b) | The arms of an ac Maxwell’s bridge are arranged as follows: AB is a non-inductive resistance of 1000 ohms in parallel with a capacitor of capacitance 0.5µF, BC is a non-inductive resistance of 600 ohms, CD is an inductive impedance (unknown) and DA is a non-inductive resistance of 400 ohms. If balance is obtained, find the value of resistance and inductance of branch CD. |  |
| c) | Explain in detail the various types of transients. | ……3/- |