## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Electrical Engg./E&EE

Subject: Electrical Measurements & Instruments(EE303/4103)[Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define i) Sensitivity ii) Accuracy iii) Precision. |  |
| b) | List the various effects of electric current. Briefly explain any one of them. |  |
| c) | State the significance of damping torque in analog indicating instruments. List the methods of providing the damping torque. |  |
| d) | List the four main parts of the operating mechanism of 1 induction type energy meter. Briefly explain any one of them. |  |
| e) | List the six types of adjustments that need to be carried out in 1 induction type energy meter. |  |
| f) | State the formula and explain the principle of measurement of medium resistance using Wheatstone Bridge. |  |
| g) | State and explain the necessity of using instrument transformer in measurement. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the classification of measuring instruments. Briefly explain any four types. |  |
| b) | State the significance of controlling torque in an analog indicating type of measuring instrument. List the methods of producing controlling torque. Explain any one method with neat diagram. |  |
| c) | Explain the construction and working principle of 3-phase energy meter. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat labelled diagram explain the construction and working of PMMC type of measuring instrument. |  |
| b) | State the various errors associated with MI type of instrument and methods adopted to minimise the same. |  |
| c) | With the help of circuit diagram explain the concept and working of rectifier type of instrument using full wave rectifier. State its advantages. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of diagram explain the construction and working of dynamometer type of 1 Wattmeter. |  |
| b) | List the various types of errors associated with dynamometer type of wattmeter. Explain any four of them in brief. |  |
| c) | A 3, 500V motor load has a p.f. of 0.4. Two wattmeters are connected for measuring the input power. The sum of the two wattmeter readings is 30KW. Find the reading of each wattmeter. |  |
|  |  | …..2/- |

|  | -2- |  |
| --- | --- | --- |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat diagram explain the principle and method of measurement of low resistance using a Kelvin’s Double Bridge. |  |
| b) | With a diagram explain the construction and working principle of a series type ohmmeter. |  |
| c) | Explain the method of extension of range of wattmeter by using CT and PT for the measurement of power in 3 3 wire circuit by using 2 wattmeter method. Draw necessary diagram. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write short note on ‘shunts and multipliers’. |  |
| b) | With the help of neat diagram explain the construction and principle of power factor measurement using 1 electrodynamometer type of power factor meter. |  |
| c) | Write short note on construction and working of Megger. |  |

***********