## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Electronics Engg./EC/EI/COMP/MED/E&EE

Subject: Microprocessors(8085)-I[4142] [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw the schematic to generate 4 control signals using combinations of :RD, WR and IOM. |  |
| b) | Explain the function of instruction register and decoder. |  |
| c) | List the classification of instruction set based on word size and give one example each. |  |
| d) | Identify the memory locations that are cleared by the instruction below: MVI B, 00H MVI C, 00H LXI H, 2075H MOV M, B INX B DCX H MOV M, B |  |
| e) | Explain the function of: i) DAD ii) SHLD. |  |
| f) | If SP=5888H, after three consecutive PUSH instructions what will be the content of SP. Also explain function of PUSH instruction. |  |
| g) | Describe the use of EI and DI. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw flag register and explain the function of each flag bit. |  |
| b) | Draw the schematic diagram of latching the lower order bus of µP-8085 using ALE signal and explain demultiplexing of Bus AD7 – AD0. |  |
| c) | Illustrate the contents of registers (A, H, L), flags (S, Z, CY) and memory location (7075H) as the following instructions are executed. LXI H, 7055H MVI M, 80H MVI A, AOH ADD M INX H STA 7075 HLT |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write instructions to follow algorithm below: |  |
|  | i) Load F085H and 1066H in register pairs B & D |  |
|  | ii) Increment register pair D and decrement register pair B |  |
|  | iii) Transfer the content of register B to register H, register C to register L and register D to Accumulator |  |
|  | iv) Exchange the content of HL and DE register pair |  |
|  | v) Store the content of accumulator indirectly to memory location specified by register pair BC |  |
|  | vi) Halt the program |  |
| b) | Explain the programming technique used by µP-8085 to repeat specific tasks. Use flowcharts. |  |
| c) | Write a program to complement 5 data bytes stored from location 7450H. Store the complemented result in memory locations starting at 7800H. | ……2/- |
|  | -2- |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Ten data bytes are stored from memory location 7075H. Write a program to copy the bytes with odd parity in a block starting at 7080H. |  |
| b) | Briefly explain the classification of µP-8085 interrupts. |  |
| c) | Write a program to turn a light ON and OFF 5 every seconds. Use data bit D7 to operate light. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is stack? Explain the loading and retrieval of data in stack. |  |
| b) | Describe the use of RIM instruction. Also draw the interpretation of accumulator bit pattern for RIM instruction. |  |
| c) | Explain nesting of subroutines and also briefly explain instructions used to CALL subroutines. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Write a note on RST instruction. |  |
| b) | Briefly describe steps involved in the interrupt process. |  |
| c) | Interface a 256 x 8 memory chip to µP-8085 such that the address range is F000H to F0FFH. (Use decoder logic) |  |
| d) | Use an example and illustrate the delay calculations using a loop within loop technique. |  |

***********