## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Materials & Processes(4238) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the properties and applications of high carbon steel. |  |
| b) | Give reasons for alloying in steels. |  |
| c) | What is malleable cast iron? Where is it used? |  |
| d) | How can steel be identified using magnetic properties, weight and colour? |  |
| e) | Explain over-stressing technique of reducing welding stresses. |  |
| f) | Explain induction method of heat treatment of steel. |  |
| g) | What is re-crystallisation temperature? |  |
| h) | What are refractory materials? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw iron-carbon equilibrium diagram (steel portion up to 2% C) and name all the structures. |  |
| b) | What influence do various constituents have on steel? |  |
| c) | Explain any two types of stainless steel giving their properties and applications. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the methods of identifying the percentage of constituents. |  |
| b) | What is weld cracking? Explain the factors influencing corrosion resistance of welded joints. |  |
| c) | Explain the following: i) Weld metal zone ii) Heat affected zone. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Why is stress relief of weldment required? What is peening? |  |
| b) | What are the effects of absorption of following gases on weldments: i) Hydrogen ii) Nitrogen iii) Oxygen. |  |
| c) | What is tempering? Explain any two methods of tempering. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is the effect of time and temperature of ageing treatment on the mechanical properties? |  |
| b) | What is strain hardening? Give its applications. |  |
| c) | Explain the various factors affecting good results in heat treatment of steel. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Steel making |  |
| b) | Alloy structural steel |  |
| c) | Grey cast iron |  |
| d) | Corrosion of welds |  |
| e) | Hot working processes |  |
| f) | Insulating materials |  |

***********