## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Civil Engg./CT/M

Subject: Hydraulics(CE405)/(4025) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the terms: Density, Specific Gravity and Specific Weight. |  |
| b) | Show the relationship between atmospheric pressure, gauge pressure, vacuum pressure and absolute pressure with help of diagram. |  |
| c) | State Bernoulli’s theorem. State its limitations. |  |
| d) | Distinguish clearly between an Orifice and a Mouth piece. |  |
| e) | Define Reynold’s Number. State its significance. |  |
| f) | What is meant by ‘Hydraulic Jump’ in an open channel? |  |
| g) | State the laws of Fluid Friction. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the principle of working of a Bourdon’s Pressure Gauge. |  |
| b) | A circular plate of 2.5m diameter is vertically immersed in water so that its centre is 4m below the water surface. Find the total pressure on the plate and the point where it acts. |  |
| c) | A differential manometer connected at two points A and B at the same level in a pipe containing oil of specific gravity 0.8, shows difference in mercury level at 90mm. Draw a neat sketch and determine the difference in pressure at the two points. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with the help of neat sketch principle and working of a Venturimeter. |  |
| b) | Water flows through a circular orifice of 0.032m diameter. The head of water above the centre of orifice is 0.8m. The jet of water falls 0.03m at a horizontal distance of 0.3m from Vena Contracta. The discharge from orifice is 0.00175 m3/sec. Find the Hydraulic coefficients Cc, Cv and Cd. |  |
| c) | A submarine fitted with a pitot tube moves horizontally in sea. Its axis is 12m below the surface of water. The pitot tube is fixed infront of a submarine and along its axis which is connected to two limbs of u-tube containing mercury, the reading of which is found to be 200mm. Find the speed of the submarine. Take specific gravity of sea water as 1.025. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A 40m long weir is divided into 12 equal bays by vertical posts each 0.6m wide. Using Francis formula calculate the discharge over the weir if the head over the crests is 1.2m and velocity of approach is 2m/s. |  |
| b) | Define the following and give one practical example for each: - Steady flow ii) Uniform flow iii) Laminar flow |  |
| c) | A triangular notch with vertex angle 60 is used to determine discharge through a channel. The head of water over the notch is 350mm. If the coefficient of discharge for the notch is 0.63, calculate the discharge in the channel. | ……..2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | An old pipe of 1m, 1.2m, 0.8m diameter laid in series have lengths 800m, 1000m and 1500m respectively. This pipe line is to be replaced by single pipe of uniform diameter with same total length. Determine the equivalent pipe diameter if the friction factor of all pipes is same. |  |
| b) | Define open channels. Classify open channels giving examples. |  |
| c) | A rectangular channel 4m wide and 2m deep has a bed slope of 1 in 3600. Using Bazins formula find the discharge through the channel if K=0.83. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Hydraulic Ram |  |
| b) | Working of Reciprocating pump |  |
| c) | Priming of Centrifugal pump |  |
| d) | Hydraulic Accumulator |  |
| e) | Orifices and Mouth pieces |  |

***********