## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Mining Engg.

Subject: Mine Development(MN301) [Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define non-metallic minerals. Give 3 examples. |  |
| b) | What are the factors affecting stripping ratio? |  |
| c) | Differentiate between an adit and an incline. |  |
| d) | Classify coal seam based on Gassiness. |  |
| e) | Mention the factors affecting the level interval. |  |
| f) | List down the unique features of mineral industry. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Give brief note on any five mineral resources produced in India. |  |
| b) | Explain the principles of development of an underground metal mine. |  |
| c) | i) Draw a neat sketch of opening an inclined deposit. ii) Give any five differences between underground coal mining and metal mining. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the different types of box cuts? Explain any one with a neat sketch. |  |
| b) | Explain the factors affecting the bench height and the bench width of an opencast mine. |  |
| c) | i) What are the cycles of operation involved in opencast mining? ii) Give the advantages and disadvantages of opencast mining. |  |
| Q.No.4. | Answer the following Questions: |  |
| a) | Explain the different operations involved in construction of a drift/incline with relevant sketches. | (8) |
| b) | What are the factors considered in selection of tunnel? | (4) |
|  | ### OR |  |
| b) | Write short notes on Tunnel Boring machine. | (4) |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch of a panel/district showing the conveyor belt arrangement. Explain the same. |  |
| b) | (i) Based on what factors does the size of Panel is selected? (ii) Differentiate between development by solid blasting and coal cutting machine. |  |
| c) | Draw a neat sketch of longwall retreating panel and mention the elements. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Give the classification of surface mining. |  |
| b) | Differentiate between a circular shaft and rectangular shaft. |  |
| c) | What type of mine entry is suitable for high production? Why? |  |
| d) | Give comparison between Bord and pillar mining and Longwall mining. |  |

***********