## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Mechanical Engg./A

Subject: Hydraulic Machinery(ME404)/(4053) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | In an experiment, the weight of 2.5m3 of a certain liquid was found to be 18.75KN. Find the specific weight of the liquid. Also find its density. |  |
| b) | Define specific weight and specific gravity of water. |  |
| c) | What do you mean by Absolute Pressure? |  |
| d) | What do you understand by turbulent flow? |  |
| e) | What is hydraulic gradient line and total energy line? |  |
| f) | What are the functions of air vessel? |  |
| g) | What are the applications of gear pump? |  |
| h) | Explain cavitation. (only for students of Revised scheme) |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain the working of an inverted Ú’ tube manometer. |  |
| b) | A simple manometer containing mercury is used to measure the pressure of water flowing in a pipeline. The mercury level in the open tube is 60mm higher than that on the left tube. If the height of water in the left tube is 50mm, determine the pressure in the pipe in terms of head of water. |  |
| c) | A rectangular plate 2m wide and 3m deep is immersed in water in such a way that its plane makes an angle of 30 with the water surface and its upper edge is 1.5m below the water surface. Calculate the total pressure and the position of centre of pressure. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | An isosceles triangles plate 3m high and 2m base is immersed vertically in water in such a way that its apex is below the base and at a distance of 4m below the water surface. Find the total pressure and position of the centre of pressure. |  |
| b) | A venturimeter is to be fitted to a 250mm diameter pipe, in which the maximum flow is 7200 lit/min and the pressure head is 6 metres of water. What is the minimum diameter of throat, so that there is no negative head in it? |  |
| c) | A small circular orifice 10mm diameter is discharging water under a constant head of 400mm. What will be the discharge through the orifice in m3/s, if cd=0.6. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State and explain laws of Fluid friction |  |
|  | only for students of Revised scheme: |  |
| b) | Explain the construction and working of Kaplan turbine with neat sketch. |  |
| c) | With neat sketch explain the governing of Pelton wheel. |  |
|  | only for students of Old scheme: |  |
| b) | Explain with neat sketch construction of hydraulic jack. |  |
| c) | Explain the working of hydraulic press with help of a neat sketch. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With help of a neat sketch explain construction and working of a centrifugal pump. |  |
| b) | Explain multistage centrifugal pumps. |  |
| c) | Explain with neat sketch working of a reciprocating pump. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the various faults and remedies of centrifugal pump? |  |
| b) | Explain with neat sketch the construction and working of vane pump. |  |
| c) | A horizontal pipe of 180mm diameter suddenly reduces its diameter to 120mm. If water flows in the pipe with a velocity of 2m/s, find the loss of head due to sudden contraction and the discharge through the pipe. |  |

***********