## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Automobile Engg.

Subject: Machine Tools & Production Process-II(4060) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw a neat labelled sketch of sensitive drilling machine. |  |
| b) | Give classification of boring machine. |  |
| c) | What are the functions of Ram and Saddle in a Shaper? |  |
| d) | Give at least three differences between Puncher Slotter and Precision Slotter. |  |
| e) | Differentiate between Shaper and Planer. |  |
| f) | List down the different operations performed on a milling machine. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the different devices used for holding and rotating milling cutters. |  |
| b) | With a neat sketch, explain plain milling and face milling operation. |  |
| c) | Explain the difference between up milling and down milling with neat sketches. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the crank and slotted link mechanism used in a Shaper. |  |
| b) | Explain machining of horizontal surface using Shaper. |  |
| c) | Draw a neat labelled sketch of a standard Shaper and give the function of the cross rail. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain any two types of milling cutters with a neat sketch. |  |
| b) | List down the various drilling machine operations and explain any two with neat sketches. |  |
| c) | What is trepanning? Calculate the machining time in drilling machine, given the following data: A 20mm diameter drill is used for drilling a hole in a mild steel block. The total length of travel of drill is 350mm, feed per revolution is 0.4mm and cutting speed is 25m per minute. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the different equipments used for mounting cutters in a horizontal boring machine. |  |
| b) | List the various work holding devices in a Planer. Describe the special features of each. |  |
| c) | Explain the machining of flat surfaces on a Slotting machine. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Work holding devices used on a drilling machine |  |
| b) | Size of horizontal boring machine |  |
| c) | Planer operations |  |
| d) | Safety precautions while working on a Slotter |  |
| e) | Form milling operation |  |

***********