## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Shipbuilding Engg.

Subject: Basic Ship Theory-I(SH301)/(4331) [Old/Revised]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short notes on any three | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Bulk carrier |  |
| b) | Moulded Breadth and Length between perpendiculars |  |
| c) | Displacement |  |
| d) | Body Plan |  |
| e) | Righting lever |  |
| f) | Block Co-efficient |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write the salient features of i) Oil tanker and ii) Catamaran. |  |
| b) | A ship has displacement 2000 tonnes and KG=4m. She loads 1500 tonnes of cargo (KG=6m), 3500 tonnes of cargo (KG=5m) and 1520 tonnes of cargo (KG=1m). She then discharges 2000 tonnes of cargo (KG=2.5m) and consumes 900 tonnes of oil fuel (KG=0.5m) during the voyage. Find the final KG on arrival at the port of destination. |  |
| c) | A ship 128 metres long has a maximum beam of 20 metres at the waterline and co-efficient of fineness of 0.85. Calculate the TPC at this draft. |  |
| Q.No.3. | The breadths at the load water plane of a ship 90 metres long measured at equal intervals from forward are as follows: 0, 3.96, 8.53, 11.58, 12.19, 12.5, 11.58, 5.18, 3.44, and 0.30m respectively. If the load draft is 5 metres and the block co-efficient is 0.6, find the water plane area and the position of the centre of floatation from amidships. | (12) |
| Q.No.4. | Answer the following Questions: | 2 x 6 = 12 |
| a) | During the course of an inclining experiment in a ship of 4000 tonnes displacement, it was found that, when a mass of 12 tonnes was moved transversely across the deck, it caused a deflection of 75mm in a plumb line which was suspended from the point 7.5m above the baten. Km=10.2m and Kg=7m. Find the distance through which the mass was moved. (only for students of revised scheme) |  |
| a) | Explain the GZ curve of stability. (only for students of old scheme) |  |
| b) | A box shaped vessel 75m long, 12m beam and 7m deep is floating on an even keel at 6m draft. Calculate the KM. |  |
| Q.No.5. | Answer any one of the following Questions: | (12) |
| a) | A box shaped vessel 40m x 6m x 3m is floating in salt water on an even keel at 2m draft F and A. Find the new drafts if a weight of 35 tonnes is discharged from a position 6m from forward. MCT1cm = 8.4 tonnes-m -2- | ……2/- |
| b) | A ship of length 140m displaces 13492tonnes when floating in sea water and has a water plane defined by the following half ordinates. Stn; AP; 1; 2; 3; 4; 5; 6; 7; 8; 9; FP. Half ordinate (m); 2.5; 6.4; 8.5; 9.4; 9.6; 9.6; 9.5; 8.4; 6.5; 3.2; 0. Calculate i) The area of water plane ii)LCF iii) The second moment of area about centerline. |  |
| Q.No.6. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Explain the following terms i) Bonjean curves ii) MCT1cm. |  |
| b) | Three consecutive ordinates in a ship’s water plane are 6.3, 3.35 and 0.75m respectively. The common interval is 6m. Find the area contained between the last two ordinates. |  |

***********