## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Mining Engg.

Subject: Surface Mining(MN401)/(4298) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the objectives of feasibility study? |  |
| b) | What is scarification? Mention the methods. |  |
| c) | Discuss the factors affecting the selection of excavating machine. |  |
| d) | What are the different means of storage of minerals? |  |
| e) | Mention the stabilisation techniques of dump slopes. |  |
| f) | What are the different ways of truck positioning at faces? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Enumerate the factors affecting bench parameters. |  |
| b) | Explain different steps involved in planning process. |  |
| c) | Describe the construction process of dumps by shovel. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the different methods of ripping. |  |
| b) | Name the drill bits used in rotary drilling. Based on what factors drill bits are selected? |  |
| c) | Discuss the specific features of “Bulk Explosives”. 1000m3 of coal is blasted with 500 Kgs of explosive. Find powder factor. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch of overcasting by a power shovel and explain important features of layout. |  |
| b) | What are the factors affecting the dragline production? Explain the type of cuts made by dragline. |  |
| c) | Explain the constructional details of wheel of BWE. How is the capacity calculated? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A 6m3 shovel working in a coal bench of height 7m with a cycle time of 50sec. The mine works in two shifts & the shovel cut width of 10m. Calculate the bench advance per day. Assume other data, if required. |  |
| b) | What are the applicable conditions of a dumper? Give a brief note on types of dumper. |  |
| c) | How do you reduce the spillage in a belt conveyer? How do you calculate the carrying capacity of belt conveyor? |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | What are the different cycles of operations in surface mining? |  |
| b) | Explain the specific features of Deck blasting. |  |
| c) | Enumerate the factors affecting the performance of wheel loader. |  |
| d) | Explain the layers in construction of Haul road. |  |

***********