## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Electronics Engg./EC/EI/COMP/MED/E/E&EE

Subject: Basic Electronics(4132) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw the atomic structure of silicon and Aluminium. |  |
| b) | Write a short note on avalanche breakdown. |  |
| c) | Draw CE configuration of transistor. What is input resistance and output resistance? |  |
| d) | Define PIV of a diode. What is PIV of a half wave rectifier? |  |
| e) | What are the applications of JFET and MOSFET? |  |
| f) | Compare Vacuum tubes and transistor. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain PN Junction diode in unbiased condition. |  |
| b) | Explain the construction and operation of full wave centre tap rectifier. Draw the necessary waveform. |  |
| c) | Write a note on conductors, Insulators and semiconductors. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the common base input and output characteristics. |  |
| b) | What is the requirement of a biasing circuit? Why fixed bias circuit is seldom used? |  |
| c) | Calculate the minimum and maximum values of emitter current for the fixed bias circuit. Also calculate the corresponding values of collector to emitter voltage. The transistor used in the circuit is a germanium transistor. Vcc = 6V, RB=10KΩ, RC=50Ω, (50 <  <200), RE=100Ω |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the current relations in CE configuration? How β of the transistor is related to ? |  |
| b) | Explain the construction and operation of N channel JFET and plot drain characteristic. |  |
| c) | Explain the construction and operation of E only MOSFET. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction, operation and plate characteristic of Vacuum triode. |  |
| b) | Explain in brief various types of electron emission. |  |
| c) | Draw and explain the three configurations of transistor and compare them. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | i) Explain the function of all the three regions of transistor. Draw the symbol of NPN transistor. |  |
|  | ii) Define rectification efficiency. What is rectification efficiency for a half wave rectifier? |  |
| b) | Explain rectifier with capacitor filter. Draw necessary waveforms. |  |
| c) | Explain VI characteristics of PN junction diode. |  |

***********