## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Civil Engg.

Subject: Design of Structures-II(Steel) [CE602] [Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of IS800-2007 code book, Steel tables is permissible.

| Q.No.1. | Answer any three of the following Questions: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Describe how the rolled steel sections are classified into different classes. |  |
| b) | With the aid of neat sketches explain the different types of welds, stating the suitability of each type. |  |
| c) | Determine the plastic section modules about strong and weak axes for ISMC 300@ 351.2 N/m. |  |
| d) | List the various steps involved in design of column section and state the parameters which affect the strength of compression member. |  |
| e) | Explain the phenomenon of “Web crippling” and necessary precautions needed. |  |
| Q.No.2. | Answer the following Questions: |  |
| a) | Design a suitable unequal single angle section to carry a tensile force of 200KN. Also design the welded end connection to connect the member to a gusset plate of 10mm thick. | (9) |
| b) | Determine the tensile strength of a tie member ISA 90 x 60 x 8 connected by its longer leg to a gusset plate of 10mm thick, using 4mm size weld as shown in fig.1. Take fy = 250MPa. | (6) |
|  | ### OR |  |
| c) | Determine the block shear strength of the welded tension member between A and B. Plate A = 200x10, Plate B = 300x10. Steel is Fe410 grade with fy = 250MPa. Refer Fig.2 | (6) |
| Q.No.3. | Answer the following Questions: |  |
| a) | Design the compression member adopting double angel equal sections connected back to back one on each side of gusset plate of 8mm thick. The member is continuous with c/c length of 2.25m. The factored axial load on the member is 210KN. Also design suitable welded end connection. Take Fe410 with fy = 250MPa. | (9) |
| b) | Determine the design axial load on the column section ISMB 400@ 604.2 N/m. The height of column is 4m and is restrained in direction and position at both ends. Use steel Fe410 grade with fy = 250MPa. | (6) |
|  | ### OR |  |
| c) | Design a discontinuous strut adopting single angle equal section. The service load on the strut is 75KN and length of member is 2.0m. Take fy=250MPa. | (6) |
| Q.No.4. | Answer the following Questions: |  |
| a) | A simply supported beam carries a service load of 20KN/m (inclusive of self weight). The effective span of the beam is 5m. Design the beam and carryout check for shear capacity, bending strength and deflection for the section adopted. Steel Fe410 grade with fy=250MPa | (9) ……2/- |
|  | -2- |  |
| b) | Define the laterally, retrained beam and explain the design procedure involved in such beams. | (6) |
|  | ### OR |  |
| c) | An ISMB 250@ 365.9 N/m is used as beam over an effective simply supported span of 5m. Determine the service load the beam can support over the entire span. The steel is Fe410 with fy =250MPa. | (6) |
| Q.No.5. | Answer the following Questions: |  |
| a) | Design a channel section purlin provided over the roof truss to carry A.C. sheet roof covering weighting 150 N/m2. The trusses are spaced at 5m c/c and purlins are place at 1.35m c/c on trusses. The intensity of wind pressure in the region is 2KN/m2. The slope of truss is 30. Take Fe410 steel with fy=250MPa. | (9) |
| b) | An angle strut in a roof truss consists of ISA 150 x 150 x 10. If the length of the strut is 2.5m and connected at the end by means of weld to gusset plate of 8mm thick, determine the compressive capacity of the member. | (6) |
|  | ### OR |  |
| c) | An ISA 125 x 75 x 10 is used as a tie member in a roof truss. The longer leg of the member is connected to gusset plate of 10mm thick by welds. If the steel is Fe410 grade, determine the tensile capacity of the member. | (6) |