## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Automobile Engg.

Subject: Thermal Engineering(4052) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Mollier chart is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is a fire tube boiler? Give three examples. |  |
| b) | What is a mixed flow turbine? |  |
| c) | Define Brake thermal efficiency and write its mathematical expression. |  |
| d) | When is multi stage compression used for air? What are its advantages? |  |
| e) | What is a black body? |  |
| f) | Explain heat transfer by radiation. |  |
| g) | How does a surface condenser work? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch of a locomotive boiler and label any five parts. |  |
| b) | With the help of a neat sketch, explain the construction and working of a dead weight safety valve. |  |
| c) | What are the advantages of using an economizer? Explain the construction of an economizer with a neat sketch. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch of throttling calorimeter and explain its uses in determination of quality of steam. |  |
| b) | Using Mollier chart, find enthalpy drop and final condition of steam when it is expanded isentropically from an initial pressure of 40 bar and 380C to a pressure of 1 bar. |  |
| c) | Give five points of difference between impulse turbine and reaction turbine. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working of diesel cycle with the help of the theoretical indicator (P-V) diagram. |  |
| b) | Give five points of difference between two stroke and four stroke cycle IC engines. |  |
| c) | Draw a diagram and explain the working of an air-compressor and mention any three uses of compressed air. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A metal pipe having an external diameter of 160mm carries steam at 200ºC. The pipe is covered by a layer of 25mm thick insulating material whose conductivity is 0.21 W/mk. If the outer surface is at 100ºC, find the amount of heat lost per meter length per minute. |  |
| b) | Mention any six points of comparison between jet and surface condenser. |  |
| c) | With the help of a neat sketch, explain the working of a counter flow jet condenser. |  |
|  |  | …..2/- |
|  | -2- |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Discuss CO2 as a refrigerant. |  |
| b) | Explain vapour compression refrigeration system. |  |
| c) | State any five properties of a good refrigerant. |  |
| d) | Define co-efficient of performance of a refrigerator. |  |

***********