## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Shipbuilding Engg.

Subject: Ship Drawing & Calculations(SH603) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) Q.No.1 is compulsory. Answer any 5 from the remaining questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short notes on any three: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Dynamic stability |  |
| b) | Floodable length and subdivision |  |
| c) | Types of Bulkheads |  |
| d) | Longitudinal framing system |  |
| Q.No.2. | From the cross curves of stability the following information has been obtained for a ship of displacement 40,000 tonnes and KG=10 metres. Angle of heel; 0; 5; 10; 15; 20; 30; 45; 60; 75; 90. GZ(m); 0; 0.9; 1.92; 3.11; 4.25; 6.3; 8.44; 9.39; 9.29; 8.5. Determine: i) Metacentric height (GM) ii) Dynamic Stability at 15. | (12) |
| Q.No.3. | A box shaped vessel 80m long, 10m wide and 5m deep is floating in salt water on an even keel at a draft of 4m. Find the new drafts if a forward compartment 5m long is bilged. KG=2m. | (12) |
| Q.No.4. | An angle stiffener 150x150x10mm is welded to the deck plate of thickness 12mm. Determine the section modulus of the angle section alongwith the deck plating of width 300mm. | (12) |
| Q.No.5. | Answer the following Questions: |  |
| a) | Draw a neat sketch to show how an upper end and lower end of corrugated bulkhead is connected to ship structure. | (4) |
| b) | Explain the procedure to obtain cross-curves of stability. How statical stability curve is obtained from cross curves of stability? | (8) |
| Q.No.6. | A uniform box shaped barge 40mx12m beam is divided into four cargo compartments each 10m long. The barge is loaded with 600 tonnes of iron ore stowed as follows: No. 01 hold – 135 tonnes, No. 02 hold – 165 tonnes No. 03 hold – 165 tonnes, No. 04 hold – 135 tonnes. The loaded barge floats in fresh water on an even keel draught of 1.75m. Construct the curve of shearing force and bending moment for this condition and also find the position and value of the maximum bending moment for the still water condition and state its type. | (12) |
| Q.No.7. | Answer the following: |  |
| a) | With a neat sketch explain the procedure to find the bending moment exerted in a ship shaped vessel. | (10) |
| b) | How stress exerted in the vessel can be reduced to bring it in the safe zone? | (02) |

***********