## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Mechanical Engg./E/FT

Subject: Refrigeration & Air Conditioning(ME502)/(5034) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Psychrometric charts is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define ‘Refrigeration effect’ and ‘COP’. |  |
| b) | What are the desirable properties of an ideal refrigerant? |  |
| c) | Draw T-S and P-h diagram for the refrigerant being dry and saturated before compression and the liquid refrigerant is under cooled after condensation. |  |
| d) | List the types of evaporators used in Refrigeration systems and give their advantages over each other. |  |
| e) | Define following: - Psychrometry - Dew point temperature - Saturated air |  |
| f) | What do you mean by sensible heating and sensible cooling? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat sketch explain construction and working of Electrolux refrigerator. |  |
| b) | List the parts of vapour compression system and explain the function of each part in brief. |  |
| c) | An ammonia refrigerator produces 15 tons of ice at 0°C in a day. The temperature range of the working cycle is 25°C and -15°C. The ammonia vapour is dry and saturated at the end of compression. Assume actual COP is 55% of theoretical. Calculate the power required to drive the compressor and mass flow rate in Kg/min. |  |
|  | Take latent heat of ice = 335 KJ/Kg and Cp (water) = 4.2 KJ/Kg°C. Use Ammonia table. Temperature °C; Specific enthalpy (KJ/Kg); Specific entropy (KJ/KgK). Liquid; Vapour; Liquid; Vapour. 25; 380.74; 1319.21; 0.3473; 4.4894. -15; -54.56; 1304.99; -0.2134; 5.0585. |  |
| d) | With a neat sketch explain Temperature concentration diagram for ammonia absorption system. (only for students of old scheme) |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain construction and working of ‘Centrifugal Compressor’. |  |
| b) | Compare Air cooled, Water cooled and Evaporative type of condensers. (Give atleast 6 points) |  |
| c) | When is dehumidification and humidification of air necessary? How it is achieved? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the following psychrometric processes and represent the processes on psychrometric chart: i) Cooling with adiabatic humidification ii) Adiabatic mixing of two air streams with mathematical expression. | …….2/- |
|  | -2- |  |
| b) | With a neat sketch explain any one type of psychrometer used for measurement of WBT and DBT of air. |  |
| c) | Atmospheric air at a dry bulb temperature of 16°C and 25% relative humidity passes through a furnace and then through a humidifier, in such a way that the final dry bulb temperature is 30° and 50% relative humidity. Find the heat and moisture added to the air. Also, determine the sensible heat factor of the process. (Represent the points on psychrometric chart) |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the functions of control systems in A/C plants? Explain any one type of temperature controller. |  |
| b) | Discuss the requirements of a good room air distribution system. (atleast 6 points) |  |
| c) | Explain: - One tonne of refrigeration - Effect of Refrigerants on Environment |  |
| d) | List the types of Air filters used in Air conditioning and explain the filter which is used for fine filtering of air with a neat sketch. (only for students of old scheme) |  |
| Q.No.6. | Write short note on any four of the following: | 4 x 3 = 12 |
| a) | Flooded type evaporator |  |
| b) | Applications of Vapour Absorption System |  |
| c) | Air handling unit (AHU) |  |
| d) | Sling Pschrometer |  |
| e) | Types of supply air outlets |  |
|  | (only for students of old scheme) |  |
| f) | Storage type v/s Instantaneous type water cooler |  |
| g) | Ice Plant |  |
| h) | Cold storage |  |
| i) | Duct design methods |  |
| j) | Solenoid valve |  |

***********