## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Medical Electronics

Subject: Medical Equipment Maintenance(MX511) [Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the convergent method of troubleshooting. |  |
| b) | Briefly explain acceptance test and calibration. |  |
| c) | Discuss about equipment service options. |  |
| d) | List three applications of ELCB and MCB each. |  |
| e) | With a neat diagram explain shell type transformer. |  |
| f) | What are the daily and weekly checks the user needs to perform to maintain sterilizers? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What do you mean by failure rate? Explain with an example. |  |
| b) | Explain troubleshooting process. |  |
| c) | What are the causes of equipment failure? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a note on maintenance organisation. |  |
| b) | Explain the prioritization procedure for purchase of spare parts requirement. |  |
| c) | Write short note on essentials of a good equipment management program. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What do you mean by earthing? Explain system earthing and equipment earthing in brief. |  |
| b) | Explain with neat diagram plate earthing electrode. |  |
| c) | Explain the construction and working principle of MCB. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat diagram the working principle of DC motors. |  |
| b) | Discuss about the reversal of rotation of DC motor. |  |
| c) | Explain the construction of PMDC motor with its advantages and disadvantages. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the faults, causes and solutions while troubleshooting ECG machines. |  |
| b) | Explain the operation of an incubator with a neat diagram. |  |
| c) | Explain the principles of operation of Autoclaves. |  |

***********