## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Mechanical Engg./A

Subject: Thermal Engineering(ME303) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Steam Tables and Mollier’s chart is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define heat. Compare heat with work. |  |
| b) | Briefly explain ‘entropy’ of a gas and state its units. |  |
| c) | What do you understand by the term irreversibility? |  |
| d) | State and explain Gay-Lussac’s law. |  |
| e) | Explain the function of nozzles used with steam turbine and state their types. |  |
| f) | What are the sources of air leakage into the steam condenser? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define and represent the following processes on P-V and T-S diagram: i) Adiabatic process ii) Isochoric process. |  |
| b) | A piston cylinder containing air expands at a constant pressure of 150KPa from a temperature of 285K to a temperature of 550K. The mass of air is 0.05Kg. Find the heat transfer, work transfer and the change in internal energy during the process. Take Cp=1 KJ/KgK and R=0.287 KJ/KgK |  |
| c) | A quantity of air is compressed according to pv1.25 = constant. The initial temperature and pressure of the gas is 150C and 1 bar respectively. Find the work done in compressing 1Kg of air at 3 bar and the heat rejected through the walls of the cylinder. γ =1.4, R=0.287 |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the values of specific volume, enthalpy, internal energy and entropy of steam at a pressure of 40 bar and with dryness fraction 0.8? |  |
| b) | Steam is supplied to a steam turbine at a pressure of 20 bar and 230C. It is then expanded isentropically to a pressure of 1 bar. Using Mollier’s chart, find Rankine efficiency. |  |
| c) | With help of a neat sketch, explain the construction and working of a steam calorimeter. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe with a neat sketch construction and working of a Cochran boiler. |  |
| b) | Explain construction and working of a La Mont boiler, with the help of a neat sketch. |  |
| c) | What are the functions of blow off cock? Explain its construction and working with help of a neat sketch. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain the construction and working of an impulse type of a steam turbine. |  |
| b) | Describe with a neat sketch forced draught cooling tower. |  |
| c) | A heat engine in a cycle, delivering work equal to 22 x 103 Nm and absorbs heat of 95KJ. Determine the efficiency of the engine and heat transfer from the working fluid. | …..2/- |
|  | -2- |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Indian boiler Regulations |  |
| b) | Regenerative feed heating |  |
| c) | Classification of condensers |  |
| d) | Principle of energy conservation |  |

***********