## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Architectural Engg.

Subject: Building Construction-III(AE303) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Foundations of building structures are laid inside the soil after excavation, why? Give one important reason. |  |
| b) | How does a column in R.C.C. withstand loads it is subjected to. Explain with sketches |  |
| c) | Why are stairs important component of a building? Give the most important reason. |  |
| d) | What is the main difference between a door and a window? |  |
| e) | What is the main function of a frame in an opening? Explain. |  |
| f) | A traditional sloping roof was generally built in Timber, why? Give one important reason. |  |
| Q.No.2. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Sketch out a plan and section through a typical grillage foundation and label all its components. |  |
| b) | Explain setting out of foundation trenches for a laterite wall load bearing structure of 3x4m. |  |
| c) | Write short notes on: i) Shoring in foundation ii) Raft foundation. |  |
| d) | Explain strap footing and list out 2 conditions in which this type is necessitated. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain i) Centering ii) Shuttering iii) Scaffolding. |  |
| b) | Draw 2 sketches each necessary to explain and label all the components of reinforcement details of i) Column ii) Beam in R.C.C. |  |
| c) | Write short notes on: i) Proportioning of concrete ii) Curing of concrete. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Give 6 requirement of a good staircase. |  |
| b) | Explain and draw sketches of any 3 types of staircases classified based on shapes. |  |
| c) | Draw a section through a Timber staircase and explain its construction. Also label all its parts. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the plan and section through an Aluminium sliding window showing all the construction details with labels. |  |
| b) | Draw the plan and elevation of a two-panelled timber door. Label all parts giving the sizes of each component. |  |
| c) | Draw sketches of any 3 important Timber joints used in doors and window construction. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and label with sizes, a Queen post roof truss in Timber. |  |
| b) | Draw 3 details of a timber truss showing the construction of joints and fixing. Label all its parts with sizes. |  |
| c) | Explain with sketches the following roof trusses: i) truncated ii) composite iii) mansard. |  |

***********