## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Shipbuilding Engg.

Subject: Computer Applications In Ship Design(SH723) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short note on any three: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Modification of sectional area curve to shift LCB |  |
| b) | Types of freeboard |  |
| c) | Weight curve for continuous material |  |
| d) | Hull form generation using modelling software |  |
| e) | Hull components Library |  |
| Q.No.2. | What is meant by tonnage certificate? List down the uses of tonnage certificate. How is gross tonnage and net tonnage determined? | (12) |
|  | OR |  |
| Q.No.2. | A bulk carrier has a depth of 4.2m and a draught of 3.4m. The total enclosed volume is 4000m3 of which 76% is used for cargo. Calculate the gross and net tonnages. What is the net tonnage of the ship if it is now decided to carry 19 passengers in cabin? | (12) |
| Q.No.3. | Explain the need to change the LCB and prismatic coefficient by modifying the sectional area curve. How is it carried out and what is the sequence followed in modifying sectional area curve? | (12) |
| Q.No.4. | Explain the procedure to develop lines plan of hull form from sectional area curve and the load waterline. | (12) |
| Q.No.5. | Explain the complete procedure to determine the weight curve during strength calculations. | (12) |
|  | OR |  |
| Q.No.5. | The mean values for weight and buoyancy as measured at centres between each of six stations are as follows: | (12) |
|  | Stn Weight (tonnes/m) Bouyancy (tonnes/m) |  |
|  | ½ 43.51 17.99 |  |
|  | 1½ 85.62 69.64 |  |
|  | 2½ 47.98 85.96 |  |
|  | 3½ 41.31 73.97 |  |
|  | 4½ 34.65 46.98 |  |
|  | 5½ 52.11 10.66 |  |
|  | Determine the bending moment values for the ship. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List down the data needed to carry out hydrostatic calculations and intact stability calculations. |  |
| b) | Explain how damaged stability calculated in basic design. |  |
| c) | Explain how documents (drawing) generated in detail design. |  |

***********